# Module 5: Filtering Neural Signals

Duration: 60 Minutes

# Module Overview

In the previous modules, we learned how neural signals are generated, acquired, and represented in both the time and frequency domains. However, neural recordings are rarely perfect. Along with the physiological signal of interest, recordings often contain unwanted components such as baseline drift, muscle activity, power-line interference, and other sources of noise.

To extract meaningful information from these recordings, unwanted signal components must be removed while preserving the physiological information of interest. This process is known as signal filtering.

Filtering is one of the most fundamental operations in neural signal processing. The choice of filter depends on the characteristics of the recorded signal and the specific application. A filter that is appropriate for one recording modality may not be suitable for another.

In this module, we will study the principles of signal filtering, examine the most commonly used filters in neuroscience, and learn how to select an appropriate filter for different neural recording applications.

# Learning Outcomes

After completing this module, you should be able to:

- Explain why neural signals require filtering.

- Distinguish between different types of filters.

- Describe the working principle of low-pass, high-pass, band-pass, and notch filters.

- Explain the purpose of moving-average and median filters.

- Select an appropriate filter for different neural recording applications.

# Module Structure

| Section | Topic | Duration |
| --- | --- | --- |
| 5.1 | Why Signals are Filtered | 8 min |
| 5.2 | Low-pass Filtering | 10 min |
| 5.3 | High-pass Filtering | 10 min |
| 5.4 | Band-pass Filtering | 10 min |
| 5.5 | Notch Filtering | 7 min |
| 5.6 | Moving-average and Median Filtering | 8 min |
| 5.7 | Selecting a Filter for a Neural Application | 7 min |

# 5.1 Why Signals are Filtered

Duration: ~8 Minutes

## 5.1.1 Introduction

In an ideal world, every neural recording would contain only the physiological signal that we wish to study. Unfortunately, this is rarely the case.

Whether we record EEG, EMG, ECoG, or microelectrode signals, the acquired waveform usually contains not only the biological signal of interest but also unwanted electrical activity originating from various sources.

These unwanted components may arise from electrical equipment, movement of the subject, muscle activity, poor electrode contact, or the surrounding environment.

If these unwanted signals are not removed, they can make interpretation difficult and reduce the accuracy of subsequent analyses.

This is why signal filtering is an essential step in neural signal processing.

## 5.1.2 Learning Objectives

After completing this section, you should be able to:

- Explain why filtering is necessary.

- Distinguish between the signal of interest and unwanted noise.

- Understand the purpose of filtering in neural signal processing.

# 5.1.3 What is Signal Filtering?

### Definition

Signal filtering is the process of selectively removing unwanted frequency components from a recorded signal while preserving the information that is relevant for analysis.

### Lecture

Filtering does not create new information.

Instead, it improves the quality of the recorded signal by reducing components that are unlikely to contain useful physiological information.

Consider an EEG recording.

The recording contains the electrical activity generated by the brain, which is the signal we are interested in studying. However, the same recording may also contain slow baseline drift, electrical interference from nearby equipment, muscle activity from the face or neck, and power-line interference.

If all of these components remain in the recording, identifying meaningful brain activity becomes much more difficult.

Filtering helps separate the signal of interest from these unwanted components.

It is important to remember that filtering is a selective process. A properly designed filter removes only those components that are considered undesirable while preserving the physiological information needed for analysis.

### Key Definition

Signal Filtering: The process of preserving useful signal components while attenuating unwanted components.

# 5.1.4 Why Do Neural Signals Need Filtering?

Neural recordings are highly susceptible to contamination because biological signals are extremely small.

Even minor electrical disturbances can produce voltage changes that are comparable to or larger than the neural signal itself.

Some common reasons for filtering include:

- Removing slow baseline drift.

- Reducing high-frequency electrical noise.

- Eliminating power-line interference.

- Improving the signal-to-noise ratio.

- Preparing signals for feature extraction and analysis.

Filtering therefore serves as an essential preprocessing step before most signal-processing techniques are applied.

# 5.1.5 Signal Versus Noise

Before applying any filter, it is important to distinguish between two concepts.

Signal

The signal refers to the physiological activity that we want to analyse.

Examples include:

- Brain activity in EEG.

- Muscle activity in EMG.

- Local field potentials.

- Neural spikes.

Noise

Noise refers to unwanted components that interfere with the signal of interest.

Examples include:

- Electrical interference.

- Motion artefacts.

- Muscle artefacts in EEG.

- Baseline drift.

- Power-line interference.

The objective of filtering is not to remove everything except the desired waveform.

Rather, the objective is to improve the visibility of the physiological signal while preserving its important characteristics.

# 5.1.6 Important Considerations

Filtering should always be performed carefully.

A filter that is too aggressive may remove important physiological information along with the unwanted noise.

Conversely, a filter that is too weak may fail to remove sufficient interference.

Therefore, selecting an appropriate filter requires an understanding of both the signal being recorded and the type of unwanted components present in the recording.

This is why different neural recording applications often use different filtering strategies.

## 5.1.7 Summary

In this section, we introduced the concept of signal filtering.

We learned that neural recordings usually contain both useful physiological information and unwanted electrical components. Filtering improves signal quality by selectively reducing unwanted components while preserving the information relevant for analysis.

Understanding why filtering is necessary provides the foundation for studying the different types of filters introduced in the following sections.

## Transition to Section 5.2

Now that we understand why neural signals are filtered, the next question is:

How does a filter decide which parts of the signal to keep and which parts to remove?

To answer this, we begin with one of the simplest and most widely used filters in signal processing—the Low-pass Filter.

# 5.2 Low-Pass Filtering

Duration: ~10 Minutes

# 5.2.1 Introduction

In the previous section, we learned that filtering is used to improve the quality of neural recordings by reducing unwanted signal components. However, not all unwanted signals occupy the same frequency range. Some unwanted components occur at very high frequencies, while others occur at much lower frequencies.

To remove unwanted high-frequency components while preserving slower physiological activity, one of the most commonly used filters is the low-pass filter.

In this section, we will understand the operating principle of a low-pass filter, learn how it modifies a signal, and discuss its applications in neural signal processing.

# 5.2.2 Learning Objectives

After completing this section, you should be able to:

- Define a low-pass filter.

- Explain the operating principle of a low-pass filter.

- Describe the concept of cutoff frequency.

- Identify situations where low-pass filtering is useful.

- Understand the advantages and limitations of low-pass filtering.

# 5.2.3 What is a Low-Pass Filter?

### Definition

A low-pass filter is a filter that allows low-frequency components of a signal to pass while attenuating or removing high-frequency components above a selected cutoff frequency.

### Lecture

The name itself explains how the filter works.

The term low-pass means that low frequencies are allowed to pass through the filter, while higher frequencies are progressively reduced.

Imagine a gate that allows only slow-moving vehicles to pass while stopping fast-moving vehicles.

A low-pass filter behaves in a similar manner.

Instead of separating vehicles according to speed, it separates signal components according to their frequency.

Low-frequency components continue through the filter with little or no change, whereas high-frequency components are attenuated.

### Key Definition

Low-Pass Filter: A filter that preserves low-frequency components while reducing high-frequency components.

# 5.2.4 The Concept of Cutoff Frequency

A low-pass filter does not simply separate frequencies into two completely distinct groups.

Instead, it is designed around a parameter known as the cutoff frequency.

### Definition

The cutoff frequency is the frequency above which signal components begin to be attenuated by the filter.

### Lecture

Suppose a low-pass filter has a cutoff frequency of 40 Hz.

This means that frequency components below 40 Hz are allowed to pass with minimal attenuation.

As the frequency increases beyond 40 Hz, the filter gradually reduces the amplitude of those higher-frequency components.

It is important to understand that the cutoff frequency is not an abrupt boundary where signals are completely removed. Instead, attenuation increases progressively beyond the cutoff frequency.

(Insert Figure 5.2: Frequency response of a low-pass filter showing the passband, cutoff frequency, and attenuation region.)

# 5.2.5 How Does a Low-Pass Filter Affect a Signal?

Consider an EEG recording that contains both useful brain activity and unwanted high-frequency electrical noise.

When this recording passes through a low-pass filter:

- Low-frequency brain activity is preserved.

- High-frequency noise is reduced.

- The resulting waveform appears smoother and easier to interpret.

The filter therefore improves the visibility of the physiological signal by suppressing unwanted high-frequency components.

However, it is important to remember that the filter removes all frequencies above the selected cutoff, regardless of whether they represent noise or useful physiological information.

For this reason, the cutoff frequency must be selected carefully.

# 5.2.6 Applications in Neural Signal Processing

Low-pass filters are widely used in neuroscience because many physiological signals contain important information at relatively low frequencies.

Some common applications include:

### EEG

Low-pass filters are frequently used to reduce high-frequency electrical noise while preserving the frequency components relevant for EEG analysis.

### Local Field Potentials (LFP)

Since LFPs primarily contain lower-frequency activity, low-pass filtering is commonly used to separate LFPs from higher-frequency spike activity.

### EMG Envelope Extraction

After rectifying an EMG signal, a low-pass filter is often applied to obtain the smooth EMG envelope, which represents the overall level of muscle activation.

# 5.2.7 Advantages

Low-pass filtering offers several advantages.

- Reduces unwanted high-frequency noise.

- Produces smoother waveforms.

- Improves signal readability.

- Serves as an important preprocessing step before feature extraction.

# 5.2.8 Limitations

Although low-pass filtering is extremely useful, it must be applied carefully.

If the cutoff frequency is selected too low, useful physiological information may also be removed.

Excessive low-pass filtering can make the signal appear overly smooth and may distort rapidly changing neural events.

Therefore, the cutoff frequency should always be chosen according to the characteristics of the signal being analysed.

### Important Note

A low-pass filter removes high-frequency components, not necessarily noise.

If useful physiological information exists above the selected cutoff frequency, that information will also be attenuated.

This is why filter selection should always be guided by the characteristics of the biological signal rather than by convenience alone.

# 5.2.9 Summary

In this section, we introduced the low-pass filter and examined its role in neural signal processing.

We learned that a low-pass filter allows low-frequency components to pass while reducing frequencies above a selected cutoff frequency.

Low-pass filtering is widely used to suppress high-frequency interference and improve signal quality. However, careful selection of the cutoff frequency is essential to avoid removing useful physiological information.

## Transition to Section 5.3

The low-pass filter removes high-frequency components while preserving lower frequencies.

But in some situations, the opposite is required.

Certain recordings contain unwanted low-frequency components, such as baseline drift or slow movement artefacts, while the useful physiological information lies at higher frequencies.

To address this problem, we use another fundamental filter—the High-Pass Filter.

# 5.3 High-Pass Filtering

Duration: ~10 Minutes

# 5.3.1 Introduction

In the previous section, we learned that a low-pass filter preserves low-frequency components while reducing high-frequency components. However, not all unwanted signals occur at high frequencies.

Many neural recordings contain slow fluctuations that are not part of the physiological signal of interest. These slow variations may arise from electrode movement, respiration, changes in electrode impedance, or gradual shifts in the recording baseline. Such low-frequency components can obscure important neural activity and make subsequent analysis more difficult.

To reduce these unwanted slow components while preserving higher-frequency information, we use a high-pass filter.

In this section, we will examine how a high-pass filter works, understand the concept of cutoff frequency in high-pass filtering, and discuss its applications in neural signal processing.

# 5.3.2 Learning Objectives

After completing this section, you should be able to:

- Define a high-pass filter.

- Explain how a high-pass filter operates.

- Describe the role of cutoff frequency.

- Identify applications of high-pass filtering in neuroscience.

- Understand the advantages and limitations of high-pass filtering.

# 5.3.3 What is a High-Pass Filter?

### Definition

A high-pass filter is a filter that allows high-frequency components of a signal to pass while attenuating or removing low-frequency components below a selected cutoff frequency.

### Lecture

As its name suggests, a high-pass filter performs the opposite operation of a low-pass filter.

Instead of preserving slow signal variations, it removes them and allows faster signal variations to remain.

Imagine a gate that allows only fast-moving vehicles to pass while stopping slower ones.

Similarly, a high-pass filter allows high-frequency signal components to continue through the system while reducing lower-frequency components.

This makes the filter particularly useful when unwanted information exists primarily at low frequencies.

### Key Definition

High-Pass Filter: A filter that preserves high-frequency components while attenuating low-frequency components below a selected cutoff frequency.

# 5.3.4 The Concept of Cutoff Frequency

Like the low-pass filter, the high-pass filter is also defined by a cutoff frequency.

### Definition

The cutoff frequency is the frequency below which signal components begin to be attenuated by the filter.

### Lecture

Suppose a high-pass filter has a cutoff frequency of 1 Hz.

Signal components above 1 Hz are largely preserved.

Components below 1 Hz are progressively attenuated.

Again, this transition is gradual rather than abrupt. Frequencies close to the cutoff are only partially attenuated, while frequencies much lower than the cutoff experience much greater attenuation.

(Insert Figure 5.3: Frequency response of a high-pass filter showing the stopband, cutoff frequency, and passband.)

# 5.3.5 How Does a High-Pass Filter Affect a Signal?

Consider an EEG recording containing slow baseline drift caused by gradual changes in electrode contact.

Although the neural activity itself may be present, the drifting baseline makes the waveform difficult to interpret.

When a high-pass filter is applied:

- Slow baseline drift is reduced.

- The baseline becomes more stable.

- Faster neural activity becomes easier to observe.

The filter therefore improves the visibility of the physiological signal by reducing unwanted low-frequency components.

However, the filter cannot distinguish between useful and unwanted low-frequency information. Any physiological activity occurring below the selected cutoff frequency will also be attenuated.

For this reason, the cutoff frequency must always be selected carefully.

# 5.3.6 Applications in Neural Signal Processing

High-pass filters are widely used in biomedical signal processing to remove slow variations that interfere with signal interpretation.

### EEG

High-pass filters help reduce slow baseline drift caused by electrode movement or changes in electrode impedance, making EEG recordings more stable for analysis.

### EMG

EMG signals often contain low-frequency motion artefacts produced by body movement.

High-pass filtering reduces these slow artefacts while preserving the higher-frequency components associated with muscle activity.

### Spike Recordings

Extracellular recordings from microelectrodes often use high-pass filters to isolate action potentials from slower local field potentials.

Since spikes contain predominantly high-frequency components, high-pass filtering helps separate them from slower neural activity.

# 5.3.7 Advantages

High-pass filtering provides several important benefits.

- Reduces baseline drift.

- Suppresses slow movement artefacts.

- Improves baseline stability.

- Enhances the visibility of rapid physiological events.

- Serves as an important preprocessing step before spike detection and EMG analysis.

# 5.3.8 Limitations

Although high-pass filtering is useful, it should be applied with care.

If the cutoff frequency is selected too high, slow physiological components that are important for analysis may also be removed.

For example, excessively high cutoff frequencies can distort slow EEG activity or alter the shape of event-related potentials.

Therefore, the cutoff frequency should always be selected according to the characteristics of the recorded signal and the objectives of the analysis.

### Important Note

A high-pass filter removes low-frequency components, not simply noise.

If useful physiological information exists below the selected cutoff frequency, that information will also be attenuated.

Filter settings should therefore be chosen based on the characteristics of the biological signal rather than using a fixed value for every application.

# 5.3.9 Summary

In this section, we introduced the high-pass filter and examined its role in neural signal processing.

We learned that a high-pass filter preserves high-frequency components while reducing frequencies below a selected cutoff frequency.

High-pass filtering is commonly used to reduce baseline drift, suppress slow movement artefacts, and improve the visibility of rapidly changing neural activity. Like all filters, it must be configured carefully to avoid removing useful physiological information.

## Transition to Section 5.4

So far, we have studied two fundamental filters.

A low-pass filter preserves low frequencies, while a high-pass filter preserves high frequencies.

However, many neural signals occupy only a specific range of frequencies. In such situations, we may wish to remove both very low and very high frequencies while preserving only the frequencies of interest.

To accomplish this, we use a band-pass filter, which will be the focus of the next section.

# 5.4 Band-Pass Filtering

Duration: ~10 Minutes

# 5.4.1 Introduction

In the previous sections, we studied two fundamental filters.

A low-pass filter removes high-frequency components, while a high-pass filter removes low-frequency components.

However, many neural signals occupy only a specific range of frequencies. In these situations, neither a low-pass filter nor a high-pass filter alone is sufficient.

Instead, we require a filter that preserves only a selected range of frequencies while removing both lower and higher frequencies outside that range.

This type of filter is called a band-pass filter.

Band-pass filtering is one of the most commonly used preprocessing techniques in neural signal processing because many physiological signals are analysed within specific frequency bands.

# 5.4.2 Learning Objectives

After completing this section, you should be able to:

- Define a band-pass filter.

- Explain how a band-pass filter operates.

- Understand the concepts of lower and upper cutoff frequencies.

- Describe the applications of band-pass filtering in neural signal processing.

- Recognise the advantages and limitations of band-pass filtering.

# 5.4.3 What is a Band-Pass Filter?

### Definition

A band-pass filter is a filter that allows only a selected range (or band) of frequencies to pass while attenuating frequencies below and above that range.

### Lecture

As the name suggests, a band-pass filter allows only a specific band of frequencies to pass through.

Unlike a low-pass filter, which preserves everything below a cutoff frequency, or a high-pass filter, which preserves everything above a cutoff frequency, a band-pass filter preserves only the frequencies lying between two selected cutoff frequencies.

In other words, it combines the behaviour of a high-pass filter and a low-pass filter.

The high-pass component removes unwanted low-frequency signals, while the low-pass component removes unwanted high-frequency signals.

The result is a signal containing only the frequency range of interest.

### Key Definition

Band-Pass Filter: A filter that preserves frequencies within a specified range while attenuating frequencies outside that range.

# 5.4.4 Lower and Upper Cutoff Frequencies

A band-pass filter is defined by two cutoff frequencies.

### Lower Cutoff Frequency

The lower cutoff frequency determines where the filter begins to preserve the signal.

Frequencies below this value are attenuated.

### Upper Cutoff Frequency

The upper cutoff frequency determines where the filter begins to suppress higher-frequency components.

Frequencies above this value are attenuated.

### Lecture

Suppose a band-pass filter is configured with a lower cutoff frequency of 1 Hz and an upper cutoff frequency of 40 Hz.

In this case:

- Frequencies below 1 Hz are reduced.

- Frequencies between 1 Hz and 40 Hz are preserved.

- Frequencies above 40 Hz are reduced.

The resulting signal contains only the frequency components within the selected band.

(Insert Figure 5.4: Frequency response of a band-pass filter showing the lower cutoff, passband, upper cutoff, and attenuation regions.)

# 5.4.5 How Does a Band-Pass Filter Affect a Signal?

Consider an EEG recording.

The recording may contain:

- Slow baseline drift.

- Brain activity.

- High-frequency electrical noise.

If we are interested in analysing typical EEG activity between 1 Hz and 40 Hz, a band-pass filter can remove both the slow baseline drift and the unwanted high-frequency noise simultaneously.

The resulting waveform is cleaner and focuses on the frequency range most relevant for the intended analysis.

This demonstrates one of the major advantages of band-pass filtering—it removes unwanted components from both ends of the frequency spectrum while preserving the frequencies of interest.

# 5.4.6 Applications in Neural Signal Processing

Band-pass filtering is one of the most widely used preprocessing techniques in neuroscience.

### EEG

EEG recordings are commonly band-pass filtered to retain the frequency range relevant to the analysis while suppressing slow drift and high-frequency interference.

### EMG

EMG signals occupy a characteristic frequency range.

Band-pass filtering helps suppress low-frequency motion artefacts and high-frequency electrical noise while preserving muscle activity.

### Local Field Potentials (LFP)

Band-pass filters are often used to isolate specific oscillatory frequency bands for further analysis.

### Brain–Computer Interfaces (BCIs)

Many BCI systems extract information from specific neural frequency bands.

Band-pass filtering is therefore an essential preprocessing step before feature extraction and classification.

# 5.4.7 Advantages

Band-pass filtering offers several important advantages.

- Removes both low-frequency and high-frequency interference.

- Improves the signal-to-noise ratio.

- Preserves only the frequency range relevant to the analysis.

- Widely applicable across different neural recording modalities.

# 5.4.8 Limitations

Although band-pass filtering is highly effective, the choice of cutoff frequencies is critical.

If the selected frequency band is too narrow, useful physiological information may be lost.

If the frequency band is too wide, unwanted components may remain in the recording.

Therefore, the cutoff frequencies should always be selected according to the characteristics of the signal and the objectives of the analysis.

### Important Note

A band-pass filter does not automatically improve every recording.

Its effectiveness depends on selecting an appropriate frequency range for the physiological signal being studied.

Choosing incorrect cutoff frequencies may remove useful information or preserve unwanted interference.

# 5.4.9 Summary

In this section, we introduced the band-pass filter and examined its role in neural signal processing.

We learned that a band-pass filter preserves only a selected range of frequencies while reducing frequencies below the lower cutoff and above the upper cutoff.

Because many neural signals occupy characteristic frequency ranges, band-pass filtering is one of the most commonly used preprocessing techniques in EEG, EMG, LFP, and brain–computer interface applications.

## Transition to Section 5.5

So far, we have studied filters that separate signals based on broad frequency ranges.

However, some types of interference occur at one very specific frequency.

A common example is power-line interference, which appears at 50 Hz or 60 Hz, depending on the electrical power system.

Rather than removing an entire range of frequencies, we may wish to remove only this single unwanted frequency while preserving the rest of the signal.

This is the purpose of the notch filter, which we will study in the next section.

# 5.5 Notch Filtering

Duration: ~7 Minutes

# 5.5.1 Introduction

In the previous sections, we learned about low-pass, high-pass, and band-pass filters. These filters remove broad ranges of frequencies to improve the quality of neural recordings.

However, not all unwanted signals occupy a wide frequency range.

Some types of interference occur almost entirely at a single frequency. One of the most common examples is power-line interference, which appears at 50 Hz in many countries and 60 Hz in others.

Removing an entire frequency range to eliminate this interference would also remove useful physiological information. Instead, we need a filter that removes only a very narrow frequency band while leaving the rest of the signal largely unchanged.

This filter is known as the notch filter.

# 5.5.2 Learning Objectives

After completing this section, you should be able to:

- Define a notch filter.

- Explain how a notch filter works.

- Describe why notch filters are used in neural signal processing.

- Identify common applications of notch filtering.

- Understand the advantages and limitations of notch filters.

# 5.5.3 What is a Notch Filter?

### Definition

A notch filter is a filter that attenuates a very narrow range of frequencies while allowing frequencies above and below that range to pass with minimal attenuation.

### Lecture

Unlike the filters we have studied so far, a notch filter is designed to remove only a specific unwanted frequency.

Imagine listening to music while a single annoying tone is constantly present in the background.

Instead of lowering the volume of the entire recording, it would be much better to remove only that unwanted tone.

A notch filter works in exactly this way.

It targets a very narrow frequency band while preserving almost all other components of the signal.

For this reason, notch filters are particularly useful when the unwanted interference occurs at a known frequency.

### Key Definition

Notch Filter: A filter that removes a narrow band of frequencies centred around a specific frequency while preserving the remaining signal.

# 5.5.4 Why is a Notch Filter Needed?

### Lecture

One of the most common sources of interference during biomedical recordings is the electrical power supply.

Electrical devices, laboratory equipment, lighting systems, and power cables all generate electromagnetic fields.

These fields can couple into recording electrodes and appear in the recorded signal as power-line interference.

The frequency of this interference depends on the local electrical power system.

- 50 Hz in many countries.

- 60 Hz in others.

Because the interference occurs at a well-defined frequency, it can often be reduced effectively using a notch filter centred at that frequency.

Rather than removing a broad range of frequencies, the notch filter removes only the narrow frequency band associated with the electrical interference.

### Figure 5.5

Signal Spectrum → 50 Hz Peak → After Notch Filter

(Insert a frequency spectrum illustrating a sharp 50 Hz interference peak before filtering and its reduction after notch filtering.)

# 5.5.5 How Does a Notch Filter Affect a Signal?

Consider an EEG recording contaminated by 50 Hz power-line interference.

Before filtering, the recording contains both the neural signal and a strong electrical component at 50 Hz.

When a notch filter centred at 50 Hz is applied:

- The narrow frequency band around 50 Hz is attenuated.

- Most of the remaining frequency components are preserved.

- The influence of power-line interference is significantly reduced.

As a result, the neural signal becomes easier to interpret without substantially altering frequencies outside the rejected band.

# 5.5.6 Applications in Neural Signal Processing

Notch filters are commonly used whenever recordings are contaminated by electrical interference.

### EEG

Notch filters are frequently applied to reduce 50 Hz or 60 Hz power-line interference that may contaminate scalp recordings.

### EMG

Surface EMG recordings can also pick up electrical interference from nearby equipment. A notch filter helps suppress this unwanted component while preserving most of the muscle activity.

### ECoG and Local Field Potentials

Although invasive recordings generally have better signal quality, they can still be affected by power-line interference. In such cases, notch filtering is used as part of the preprocessing pipeline.

# 5.5.7 Advantages

Notch filtering offers several advantages.

- Effectively removes narrow-band interference.

- Preserves most of the useful physiological signal.

- Simple to implement.

- Widely used in biomedical signal processing.

# 5.5.8 Limitations

Although notch filters are highly effective for removing power-line interference, they should not be used unnecessarily.

If useful physiological information exists near the notch frequency, that information may also be attenuated.

Furthermore, a notch filter removes only a specific narrow frequency band. It is not designed to remove broadband noise or motion artefacts.

Therefore, notch filtering should be used only when there is clear evidence of narrow-band interference.

### Important Note

A notch filter is not a replacement for proper recording practices.

Whenever possible, power-line interference should first be minimised by ensuring good electrode contact, proper grounding, appropriate shielding, and correct equipment setup.

Filtering should be used to reduce residual interference rather than compensate for poor recording conditions.

# 5.5.9 Summary

In this section, we introduced the notch filter and examined its role in neural signal processing.

We learned that a notch filter removes a very narrow frequency band centred around a specific frequency, making it particularly useful for reducing power-line interference at 50 Hz or 60 Hz.

Because notch filters affect only a small portion of the frequency spectrum, they preserve most of the physiological signal while reducing unwanted electrical interference.

## Transition to Section 5.6

The filters we have studied so far all separate signals based on their frequency content.

However, not every filtering technique works in the frequency domain.

Some filters operate directly on the signal waveform in the time domain, making them particularly useful for reducing random fluctuations and impulsive noise.

In the next section, we will study two simple yet widely used time-domain filters: the moving-average filter and the median filter.

# 5.6 Moving-Average and Median Filtering

Duration: ~8 Minutes

# 5.6.1 Introduction

In the previous sections, we studied filters such as low-pass, high-pass, band-pass, and notch filters. These filters operate by modifying the frequency content of a signal.

However, not all filtering techniques work in the frequency domain.

Some filters operate directly on the signal in the time domain, modifying the waveform sample by sample. These filters are simple to implement and are particularly useful for reducing random fluctuations and impulsive noise.

Two of the most widely used time-domain filters are the moving-average filter and the median filter.

In this section, we will understand how these filters work, compare their characteristics, and discuss when each filter should be used.

# 5.6.2 Learning Objectives

After completing this section, you should be able to:

- Explain the principle of a moving-average filter.

- Explain the principle of a median filter.

- Compare moving-average and median filters.

- Identify situations where each filter is appropriate.

- Understand the advantages and limitations of both filters.

# 5.6.3 Moving-Average Filter

### Definition

A moving-average filter is a time-domain filter that smooths a signal by replacing each sample with the average of its neighbouring samples.

### Lecture

The moving-average filter is one of the simplest filtering techniques.

Instead of examining the frequency content of a signal, it looks directly at the waveform.

For every sample in the signal, the filter calculates the average of a small group of neighbouring samples and replaces the original value with this average.

This process is repeated across the entire signal.

As a result, rapid fluctuations become smoother while slower trends are preserved.

Because averaging reduces random variations, the moving-average filter is often used to reduce high-frequency random noise.

### Example

Suppose we calculate the average of every five consecutive samples.

If one sample contains a small random fluctuation, averaging with the neighbouring samples reduces its influence.

Repeating this process throughout the recording produces a smoother waveform.

(Insert Figure 5.6A: Original noisy signal compared with the output of a moving-average filter.)

### Advantages

- Simple to understand.

- Easy to implement.

- Produces smoother waveforms.

- Effective for reducing random high-frequency fluctuations.

### Limitations

Although the moving-average filter reduces noise, it also smooths the signal.

If the averaging window is too large, sharp physiological events may become blurred or distorted.

Therefore, the window size should always be selected carefully.

# 5.6.4 Median Filter

### Definition

A median filter is a time-domain filter that replaces each sample with the median value of its neighbouring samples instead of their average.

### Lecture

Unlike the moving-average filter, the median filter does not calculate an average.

Instead, it examines a small group of neighbouring samples, arranges them in numerical order, and selects the middle value.

This simple difference makes the median filter particularly effective for removing isolated spikes or impulsive noise.

Because extreme values have little influence on the median, the filter can remove sudden unwanted peaks without excessively smoothing the rest of the waveform.

### Example

Imagine five neighbouring samples have the following values:

5, 6, 7, 35, 8

The value 35 represents an unwanted spike.

A moving-average filter would be influenced by this large value.

A median filter, however, selects the middle value after sorting the samples:

5, 6, 7, 8, 35

The median is 7.

As a result, the unwanted spike is removed while the surrounding waveform is preserved.

(Insert Figure 5.6B: Original signal with impulsive noise compared with the output of a median filter.)

### Advantages

- Excellent for removing impulsive or spike-like noise.

- Preserves sharp transitions better than a moving-average filter.

- Less affected by extreme values.

### Limitations

The median filter is primarily designed for impulsive noise.

It is generally less effective than frequency-based filters for reducing continuous high-frequency electrical noise.

# 5.6.5 Moving-Average vs Median Filter

Although both filters operate in the time domain, they are designed for different purposes.

| Feature | Moving-Average Filter | Median Filter |
| --- | --- | --- |
| Principle | Uses the average of neighbouring samples | Uses the median of neighbouring samples |
| Best suited for | Random high-frequency fluctuations | Impulsive or spike-like noise |
| Effect on waveform | Produces a smoother waveform | Preserves edges and sharp transitions |
| Sensitivity to extreme values | High | Low |

This comparison highlights that no single filter is ideal for every situation. The choice depends on the characteristics of the recorded signal and the type of noise present.

### Important Note

Moving-average and median filters are complementary, not competing techniques.

The moving-average filter is generally preferred for reducing random fluctuations, whereas the median filter is particularly useful when isolated spikes or impulsive noise are present.

Selecting the appropriate filter always depends on the nature of the recording.

# 5.6.6 Applications in Neural Signal Processing

Both filters are widely used during signal preprocessing.

### Moving-Average Filter

Common applications include:

- Smoothing EEG waveforms.

- Estimating EMG envelopes.

- Reducing random fluctuations before feature extraction.

### Median Filter

Common applications include:

- Removing impulsive recording artefacts.

- Eliminating isolated electrical spikes.

- Cleaning signals before segmentation or classification.

# 5.6.7 Summary

In this section, we introduced two important time-domain filtering techniques.

The moving-average filter reduces random fluctuations by averaging neighbouring samples, producing a smoother signal.

The median filter replaces each sample with the median of neighbouring values, making it particularly effective for removing impulsive noise while preserving sharp waveform features.

Although both filters improve signal quality, they are designed to address different types of noise and should be selected according to the characteristics of the recorded signal.

## Transition to Section 5.7

Throughout this module, we have studied several different filtering techniques.

Each filter removes a different type of unwanted signal, and each has its own strengths and limitations.

This naturally leads to an important practical question:

How do we decide which filter is most appropriate for a particular neural recording?

In the final section of this module, we will learn how to select an appropriate filter for different neural signal processing applications.

# 5.7 Selecting a Filter for a Neural Application

Duration: ~7 Minutes

# 5.7.1 Introduction

Throughout this module, we have studied several different filtering techniques, including low-pass, high-pass, band-pass, notch, moving-average, and median filters.

Each of these filters was designed to solve a different problem.

This raises an important question.

If several filters are available, how do we decide which one should be used for a particular neural recording?

The answer depends on one simple principle:

The choice of filter should always be determined by the characteristics of the recorded signal and the objective of the analysis.

There is no single filter that is appropriate for every application. Selecting the correct filter requires understanding both the physiological signal of interest and the unwanted components present in the recording.

# 5.7.2 Learning Objectives

After completing this section, you should be able to:

- Identify the factors involved in filter selection.

- Match different filters to common neural recording problems.

- Understand why filter selection depends on the application.

- Recognise that no single filter is suitable for every recording.

# 5.7.3 Principles of Filter Selection

### Lecture

Selecting a filter is not simply a matter of choosing the most powerful filter.

Instead, the filter should remove unwanted components while preserving the physiological information required for analysis.

Before selecting a filter, several questions should be considered.

- What signal is being recorded?

- What type of noise or interference is present?

- Which frequency components contain the useful physiological information?

- Which frequency components should be removed?

Only after answering these questions can an appropriate filter be selected.

### Key Principle

Choose the filter based on the signal, not on the software.

# 5.7.4 Choosing the Appropriate Filter

The following guidelines illustrate when each filter is commonly used.

### Low-Pass Filter

Use a low-pass filter when the objective is to remove unwanted high-frequency noise while preserving slower physiological activity.

Typical applications include:

- Reducing high-frequency noise in EEG.

- Smoothing physiological waveforms.

- Extracting the EMG envelope after rectification.

### High-Pass Filter

Use a high-pass filter when the recording contains unwanted low-frequency components.

Typical applications include:

- Removing baseline drift.

- Reducing slow motion artefacts.

- Isolating spike activity from slower signals.

### Band-Pass Filter

Use a band-pass filter when the physiological signal occupies a specific frequency range.

Typical applications include:

- EEG preprocessing.

- EMG preprocessing.

- Local field potential analysis.

- Brain–computer interface applications.

### Notch Filter

Use a notch filter when the recording contains power-line interference or another unwanted narrow-band frequency.

Typical applications include:

- Removing 50 Hz interference.

- Removing 60 Hz interference.

### Moving-Average Filter

Use a moving-average filter when the objective is to reduce random fluctuations and obtain a smoother waveform.

Typical applications include:

- Signal smoothing.

- EMG envelope estimation.

- General preprocessing.

### Median Filter

Use a median filter when the recording contains isolated spikes or impulsive noise.

Typical applications include:

- Removing impulsive artefacts.

- Eliminating isolated recording spikes.

- Cleaning signals before further analysis.

# 5.7.5 Selecting Filters for Common Neural Signals

The appropriate filter also depends on the recording modality.

| Neural Signal | Common Filtering Approach |
| --- | --- |
| EEG | Band-pass filter with notch filter when required |
| EMG | Band-pass filter, moving-average filter for envelope extraction |
| Local Field Potentials | Low-pass or band-pass filter |
| Spike Recordings | High-pass filter |
| ECoG | Band-pass filter with notch filter if electrical interference is present |

This table provides general guidance.

The exact filter settings should always be selected according to the objectives of the study and the characteristics of the recorded data.

# 5.7.6 Common Mistakes During Filter Selection

Selecting an inappropriate filter can reduce the quality of the recorded signal.

Some common mistakes include:

- Applying filters without first identifying the source of the noise.

- Selecting cutoff frequencies that remove useful physiological information.

- Applying excessive filtering, resulting in distorted waveforms.

- Using a notch filter when no power-line interference is present.

- Assuming that one filter is suitable for every recording.

These mistakes highlight the importance of understanding both the signal and the purpose of the analysis before applying any filtering technique.

### Important Note

Filtering should always be performed with a clear objective.

The goal is not to produce the smoothest possible waveform.

The goal is to preserve the physiological information that is important for the intended analysis while reducing unwanted interference.

# 5.7.7 Summary

In this section, we discussed how to select an appropriate filter for different neural signal processing applications.

We learned that filter selection depends on:

- The physiological signal being recorded.

- The type of unwanted interference present.

- The frequency range containing useful information.

- The objectives of the analysis.

Rather than relying on a single filtering technique, effective neural signal processing requires selecting the filter that best matches the characteristics of the recording.

# Module 5 Recap

In this module, we introduced the fundamental concepts of signal filtering and examined the most commonly used filters in neural signal processing.

We learned that:

- Neural recordings often contain unwanted noise and interference.

- Low-pass filters preserve low-frequency components while reducing high-frequency components.

- High-pass filters preserve high-frequency components while reducing low-frequency components.

- Band-pass filters preserve only a selected frequency range.

- Notch filters remove narrow-band interference, such as power-line noise.

- Moving-average filters smooth signals by averaging neighbouring samples.

- Median filters remove impulsive noise while preserving sharp waveform features.

- The most appropriate filter depends on the recording modality, the type of interference, and the objectives of the analysis.

Understanding these filtering techniques provides the foundation for the next module, where we will study non-stationary neural signals, including the Short-Time Fourier Transform (STFT), spectrograms, wavelet analysis, and event-related potentials, which allow us to analyse how neural signals change over time.