# 2.4 Sampling and Analog-to-Digital Conversion (ADC)

Duration: ~10 Minutes

## 2.4.1 Introduction

In the previous section, we learned that neural signals are amplified and conditioned before further processing. At this stage, the signal is still an analogue signal, meaning it varies continuously with time.

However, computers cannot process continuous electrical signals directly. They require information to be represented as digital numbers.

This conversion from a continuous analogue signal to digital data is one of the most important steps in neural signal acquisition. It enables neural recordings to be stored, displayed, processed, and analysed using digital signal processing techniques.

In this section, we will learn how analogue neural signals are converted into digital data through sampling and Analog-to-Digital Conversion (ADC).

## 2.4.2 Learning Objectives

After completing this section, you should be able to:

- Differentiate between analogue and digital signals.

- Explain the concept of sampling.

- Define sampling frequency.

- Describe the role of an Analog-to-Digital Converter (ADC).

- Understand why proper sampling is important for accurate neural signal recording.

# 2.4.3 Analog and Digital Signals

### Definition

An analogue signal is a signal whose amplitude changes continuously over time.

A digital signal represents information as a sequence of discrete numerical values.

### Lecture

The electrical signals generated by neurons are naturally analogue. As neural activity changes, the voltage also changes continuously.

Although this continuous representation accurately reflects physiological activity, computers cannot directly interpret continuously varying voltages. Digital systems store and process information as numbers.

Therefore, before a neural signal can be analysed using software, it must first be converted from its analogue form into a digital representation.

This conversion is performed by the Analog-to-Digital Converter.

### Key Definition

Analogue Signal: A continuously varying electrical signal.

Digital Signal: A numerical representation of a signal obtained after analogue-to-digital conversion.

# 2.4.4 What is Sampling?

### Definition

Sampling is the process of measuring the amplitude of an analogue signal at regular intervals of time.

Each measurement is called a sample.

### Lecture

Imagine taking photographs of a moving object every second.

Instead of recording every instant continuously, you capture a series of individual images.

Sampling works in a similar manner.

Rather than recording every continuous change in voltage, the acquisition system measures the signal at equally spaced time intervals.

Each measurement represents the signal at that particular instant.

When thousands of samples are collected every second, they provide an accurate digital representation of the original analogue waveform.

### Example

Suppose an EEG recording system samples the signal 1000 times every second.

This means that every second, the system records 1000 individual voltage measurements.

These measurements collectively represent the original EEG waveform in digital form.

# 2.4.5 Sampling Frequency

### Definition

The sampling frequency, also called the sampling rate, is the number of samples collected every second.

It is measured in Hertz (Hz).

### Lecture

Sampling frequency determines how frequently the analogue signal is measured.

A sampling frequency of:

- 250 Hz means 250 samples are collected every second.

- 500 Hz means 500 samples are collected every second.

- 1000 Hz means 1000 samples are collected every second.

Higher sampling frequencies provide more detailed information about the signal because more measurements are taken over the same period of time.

However, higher sampling rates also increase data storage requirements and computational cost.

Selecting an appropriate sampling frequency therefore involves balancing recording accuracy with efficient data processing.

### Important Note

The sampling frequency must be sufficiently high to capture the important characteristics of the biological signal without losing information.

# 2.4.6 The Analog-to-Digital Converter (ADC)

### Definition

An Analog-to-Digital Converter (ADC) is an electronic component that converts an analogue electrical signal into a sequence of digital numbers.

### Lecture

The ADC serves as the bridge between the analogue world and the digital world.

After the biological signal has been amplified and conditioned, it enters the ADC.

The ADC performs two essential tasks:

- It samples the analogue signal at regular time intervals.

- It converts each sample into a numerical value that can be processed by a computer.

The resulting sequence of numbers forms the digital representation of the original neural signal.

Without an ADC, modern digital storage, visualization, and signal processing would not be possible.

### Figure 2.4

Analog Signal → Sampling → ADC → Digital Data

(Insert a diagram illustrating the conversion of a continuous analogue waveform into discrete digital samples.)

# 2.4.7 Importance of Proper Sampling

The quality of the digital recording depends greatly on how the signal is sampled.

If too few samples are collected, important variations in the neural signal may not be represented accurately.

Conversely, collecting more samples than necessary increases storage requirements and processing time without always providing additional useful information.

For this reason, selecting an appropriate sampling frequency is an important consideration during the design of every neural recording system.

### Important Note

The accuracy of later analyses—such as filtering, spectral analysis, and feature extraction—depends on the quality of the digital data produced during the sampling stage.

A poorly digitised signal cannot be fully corrected during later processing.

# 2.4.8 Summary

In this section, we introduced the process of converting analogue neural signals into digital data.

We learned that:

- Neural signals are naturally analogue.

- Computers require digital data for processing.

- Sampling measures the analogue signal at regular time intervals.

- The sampling frequency specifies how many samples are collected each second.

- The Analog-to-Digital Converter performs the conversion from analogue signals to digital numbers.

Accurate sampling is essential because every subsequent stage of digital signal processing depends on the quality of the digitised signal.

## Transition to Section 2.5

At this point, we have completed the major stages of the neural signal acquisition chain—from detecting the biological signal to converting it into digital data.

However, one final aspect remains equally important.

Because neural recordings involve electronic equipment connected to the human body, ensuring electrical safety is essential for both the participant and the recording system.

In the final section of this module, we will discuss the principles of electrical safety and isolation in neural signal acquisition.