# 5.4 Band-Pass Filtering

Duration: ~10 Minutes

# 5.4.1 Introduction

In the previous sections, we studied two fundamental filters.

A low-pass filter removes high-frequency components, while a high-pass filter removes low-frequency components.

However, many neural signals occupy only a specific range of frequencies. In these situations, neither a low-pass filter nor a high-pass filter alone is sufficient.

Instead, we require a filter that preserves only a selected range of frequencies while removing both lower and higher frequencies outside that range.

This type of filter is called a band-pass filter.

Band-pass filtering is one of the most commonly used preprocessing techniques in neural signal processing because many physiological signals are analysed within specific frequency bands.

# 5.4.2 Learning Objectives

After completing this section, you should be able to:

- Define a band-pass filter.

- Explain how a band-pass filter operates.

- Understand the concepts of lower and upper cutoff frequencies.

- Describe the applications of band-pass filtering in neural signal processing.

- Recognise the advantages and limitations of band-pass filtering.

# 5.4.3 What is a Band-Pass Filter?

### Definition

A band-pass filter is a filter that allows only a selected range (or band) of frequencies to pass while attenuating frequencies below and above that range.

### Lecture

As the name suggests, a band-pass filter allows only a specific band of frequencies to pass through.

Unlike a low-pass filter, which preserves everything below a cutoff frequency, or a high-pass filter, which preserves everything above a cutoff frequency, a band-pass filter preserves only the frequencies lying between two selected cutoff frequencies.

In other words, it combines the behaviour of a high-pass filter and a low-pass filter.

The high-pass component removes unwanted low-frequency signals, while the low-pass component removes unwanted high-frequency signals.

The result is a signal containing only the frequency range of interest.

### Key Definition

Band-Pass Filter: A filter that preserves frequencies within a specified range while attenuating frequencies outside that range.

# 5.4.4 Lower and Upper Cutoff Frequencies

A band-pass filter is defined by two cutoff frequencies.

### Lower Cutoff Frequency

The lower cutoff frequency determines where the filter begins to preserve the signal.

Frequencies below this value are attenuated.

### Upper Cutoff Frequency

The upper cutoff frequency determines where the filter begins to suppress higher-frequency components.

Frequencies above this value are attenuated.

### Lecture

Suppose a band-pass filter is configured with a lower cutoff frequency of 1 Hz and an upper cutoff frequency of 40 Hz.

In this case:

- Frequencies below 1 Hz are reduced.

- Frequencies between 1 Hz and 40 Hz are preserved.

- Frequencies above 40 Hz are reduced.

The resulting signal contains only the frequency components within the selected band.

(Insert Figure 5.4: Frequency response of a band-pass filter showing the lower cutoff, passband, upper cutoff, and attenuation regions.)

# 5.4.5 How Does a Band-Pass Filter Affect a Signal?

Consider an EEG recording.

The recording may contain:

- Slow baseline drift.

- Brain activity.

- High-frequency electrical noise.

If we are interested in analysing typical EEG activity between 1 Hz and 40 Hz, a band-pass filter can remove both the slow baseline drift and the unwanted high-frequency noise simultaneously.

The resulting waveform is cleaner and focuses on the frequency range most relevant for the intended analysis.

This demonstrates one of the major advantages of band-pass filtering—it removes unwanted components from both ends of the frequency spectrum while preserving the frequencies of interest.

# 5.4.6 Applications in Neural Signal Processing

Band-pass filtering is one of the most widely used preprocessing techniques in neuroscience.

### EEG

EEG recordings are commonly band-pass filtered to retain the frequency range relevant to the analysis while suppressing slow drift and high-frequency interference.

### EMG

EMG signals occupy a characteristic frequency range.

Band-pass filtering helps suppress low-frequency motion artefacts and high-frequency electrical noise while preserving muscle activity.

### Local Field Potentials (LFP)

Band-pass filters are often used to isolate specific oscillatory frequency bands for further analysis.

### Brain–Computer Interfaces (BCIs)

Many BCI systems extract information from specific neural frequency bands.

Band-pass filtering is therefore an essential preprocessing step before feature extraction and classification.

# 5.4.7 Advantages

Band-pass filtering offers several important advantages.

- Removes both low-frequency and high-frequency interference.

- Improves the signal-to-noise ratio.

- Preserves only the frequency range relevant to the analysis.

- Widely applicable across different neural recording modalities.

# 5.4.8 Limitations

Although band-pass filtering is highly effective, the choice of cutoff frequencies is critical.

If the selected frequency band is too narrow, useful physiological information may be lost.

If the frequency band is too wide, unwanted components may remain in the recording.

Therefore, the cutoff frequencies should always be selected according to the characteristics of the signal and the objectives of the analysis.

### Important Note

A band-pass filter does not automatically improve every recording.

Its effectiveness depends on selecting an appropriate frequency range for the physiological signal being studied.

Choosing incorrect cutoff frequencies may remove useful information or preserve unwanted interference.

# 5.4.9 Summary

In this section, we introduced the band-pass filter and examined its role in neural signal processing.

We learned that a band-pass filter preserves only a selected range of frequencies while reducing frequencies below the lower cutoff and above the upper cutoff.

Because many neural signals occupy characteristic frequency ranges, band-pass filtering is one of the most commonly used preprocessing techniques in EEG, EMG, LFP, and brain–computer interface applications.

## Transition to Section 5.5

So far, we have studied filters that separate signals based on broad frequency ranges.

However, some types of interference occur at one very specific frequency.

A common example is power-line interference, which appears at 50 Hz or 60 Hz, depending on the electrical power system.

Rather than removing an entire range of frequencies, we may wish to remove only this single unwanted frequency while preserving the rest of the signal.

This is the purpose of the notch filter, which we will study in the next section.