# 5.6 Moving-Average and Median Filtering

Duration: ~8 Minutes

# 5.6.1 Introduction

In the previous sections, we studied filters such as low-pass, high-pass, band-pass, and notch filters. These filters operate by modifying the frequency content of a signal.

However, not all filtering techniques work in the frequency domain.

Some filters operate directly on the signal in the time domain, modifying the waveform sample by sample. These filters are simple to implement and are particularly useful for reducing random fluctuations and impulsive noise.

Two of the most widely used time-domain filters are the moving-average filter and the median filter.

In this section, we will understand how these filters work, compare their characteristics, and discuss when each filter should be used.

# 5.6.2 Learning Objectives

After completing this section, you should be able to:

- Explain the principle of a moving-average filter.

- Explain the principle of a median filter.

- Compare moving-average and median filters.

- Identify situations where each filter is appropriate.

- Understand the advantages and limitations of both filters.

# 5.6.3 Moving-Average Filter

### Definition

A moving-average filter is a time-domain filter that smooths a signal by replacing each sample with the average of its neighbouring samples.

### Lecture

The moving-average filter is one of the simplest filtering techniques.

Instead of examining the frequency content of a signal, it looks directly at the waveform.

For every sample in the signal, the filter calculates the average of a small group of neighbouring samples and replaces the original value with this average.

This process is repeated across the entire signal.

As a result, rapid fluctuations become smoother while slower trends are preserved.

Because averaging reduces random variations, the moving-average filter is often used to reduce high-frequency random noise.

### Example

Suppose we calculate the average of every five consecutive samples.

If one sample contains a small random fluctuation, averaging with the neighbouring samples reduces its influence.

Repeating this process throughout the recording produces a smoother waveform.

(Insert Figure 5.6A: Original noisy signal compared with the output of a moving-average filter.)

### Advantages

- Simple to understand.

- Easy to implement.

- Produces smoother waveforms.

- Effective for reducing random high-frequency fluctuations.

### Limitations

Although the moving-average filter reduces noise, it also smooths the signal.

If the averaging window is too large, sharp physiological events may become blurred or distorted.

Therefore, the window size should always be selected carefully.

# 5.6.4 Median Filter

### Definition

A median filter is a time-domain filter that replaces each sample with the median value of its neighbouring samples instead of their average.

### Lecture

Unlike the moving-average filter, the median filter does not calculate an average.

Instead, it examines a small group of neighbouring samples, arranges them in numerical order, and selects the middle value.

This simple difference makes the median filter particularly effective for removing isolated spikes or impulsive noise.

Because extreme values have little influence on the median, the filter can remove sudden unwanted peaks without excessively smoothing the rest of the waveform.

### Example

Imagine five neighbouring samples have the following values:

5, 6, 7, 35, 8

The value 35 represents an unwanted spike.

A moving-average filter would be influenced by this large value.

A median filter, however, selects the middle value after sorting the samples:

5, 6, 7, 8, 35

The median is 7.

As a result, the unwanted spike is removed while the surrounding waveform is preserved.

(Insert Figure 5.6B: Original signal with impulsive noise compared with the output of a median filter.)

### Advantages

- Excellent for removing impulsive or spike-like noise.

- Preserves sharp transitions better than a moving-average filter.

- Less affected by extreme values.

### Limitations

The median filter is primarily designed for impulsive noise.

It is generally less effective than frequency-based filters for reducing continuous high-frequency electrical noise.

# 5.6.5 Moving-Average vs Median Filter

Although both filters operate in the time domain, they are designed for different purposes.

| Feature | Moving-Average Filter | Median Filter |
| --- | --- | --- |
| Principle | Uses the average of neighbouring samples | Uses the median of neighbouring samples |
| Best suited for | Random high-frequency fluctuations | Impulsive or spike-like noise |
| Effect on waveform | Produces a smoother waveform | Preserves edges and sharp transitions |
| Sensitivity to extreme values | High | Low |

This comparison highlights that no single filter is ideal for every situation. The choice depends on the characteristics of the recorded signal and the type of noise present.

### Important Note

Moving-average and median filters are complementary, not competing techniques.

The moving-average filter is generally preferred for reducing random fluctuations, whereas the median filter is particularly useful when isolated spikes or impulsive noise are present.

Selecting the appropriate filter always depends on the nature of the recording.

# 5.6.6 Applications in Neural Signal Processing

Both filters are widely used during signal preprocessing.

### Moving-Average Filter

Common applications include:

- Smoothing EEG waveforms.

- Estimating EMG envelopes.

- Reducing random fluctuations before feature extraction.

### Median Filter

Common applications include:

- Removing impulsive recording artefacts.

- Eliminating isolated electrical spikes.

- Cleaning signals before segmentation or classification.

# 5.6.7 Summary

In this section, we introduced two important time-domain filtering techniques.

The moving-average filter reduces random fluctuations by averaging neighbouring samples, producing a smoother signal.

The median filter replaces each sample with the median of neighbouring values, making it particularly effective for removing impulsive noise while preserving sharp waveform features.

Although both filters improve signal quality, they are designed to address different types of noise and should be selected according to the characteristics of the recorded signal.

## Transition to Section 5.7

Throughout this module, we have studied several different filtering techniques.

Each filter removes a different type of unwanted signal, and each has its own strengths and limitations.

This naturally leads to an important practical question:

How do we decide which filter is most appropriate for a particular neural recording?

In the final section of this module, we will learn how to select an appropriate filter for different neural signal processing applications.