# 6.3 Spectrograms

Duration: ~8 Minutes

# 6.3.1 Introduction

In the previous section, we introduced the Short-Time Fourier Transform (STFT) and learned how it analyses a signal using a sequence of short, overlapping time windows. Unlike the conventional Fourier Transform, STFT provides information about both time and frequency, allowing us to study signals whose characteristics change over time.

However, performing an STFT produces a large number of individual frequency spectra—one for each analysis window.

Interpreting hundreds or even thousands of separate spectra is neither practical nor intuitive.

To solve this problem, the results of the STFT are displayed as a single visual representation called a spectrogram.

A spectrogram allows us to observe how the frequency content of a signal evolves over time, making it one of the most widely used visualization tools in neural signal processing.

# 6.3.2 Learning Objectives

After completing this section, you should be able to:

Define a spectrogram.

Explain how a spectrogram is generated.

Interpret the different components of a spectrogram.

Relate a spectrogram to the original neural signal.

Describe common applications of spectrograms in neuroscience.

# 6.3.3 What is a Spectrogram?

### Definition

A spectrogram is a graphical representation of how the frequency content of a signal changes over time.

It is generated by computing the Short-Time Fourier Transform over consecutive segments of the signal.

### Lecture

Instead of displaying only one frequency spectrum, a spectrogram combines the spectra from all STFT windows into a single image.

This allows us to observe both when specific frequencies occur and how their strengths change over time.

A spectrogram transforms a complex sequence of frequency spectra into a visual map that is much easier to interpret.

For this reason, spectrograms are widely used in speech processing, biomedical engineering, and neuroscience.

### Key Definition

Spectrogram: A time–frequency representation showing how the frequency content of a signal changes over time.

# 6.3.4 Understanding a Spectrogram

A spectrogram consists of three main components.

### Time Axis

The horizontal axis represents time.

As we move from left to right, we observe how the signal evolves throughout the recording.

### Frequency Axis

The vertical axis represents frequency, usually measured in Hertz (Hz).

Lower frequencies appear near the bottom of the spectrogram, while higher frequencies appear near the top.

### Colour Intensity

The colour represents the strength or energy of each frequency component at a particular time.

Brighter or warmer colours generally indicate stronger activity.

Darker or cooler colours indicate weaker activity.

(Insert Figure 6.4: Example spectrogram with labelled time axis, frequency axis, and colour scale.)

# 6.3.5 How is a Spectrogram Generated?

A spectrogram is generated directly from the STFT.

The process consists of four steps.

### Step 1 — Divide the Signal into Short Windows

The signal is divided into a series of overlapping time segments.

### Step 2 — Compute the STFT

A Fourier Transform is performed for each window.

Each window produces one frequency spectrum.

### Step 3 — Arrange the Spectra in Time Order

The spectra are placed side by side according to the position of each analysis window.

### Step 4 — Display the Signal as an Image

The frequency spectra are represented using colour intensity, producing a two-dimensional image that simultaneously displays time, frequency, and signal strength.

# 6.3.6 Interpreting a Spectrogram

Learning to interpret a spectrogram is an important skill in neural signal processing.

Suppose we analyse an EEG recording.

If alpha activity appears only between 5 seconds and 10 seconds, the spectrogram will show increased energy around the alpha frequency range only during that time interval.

Similarly, if higher-frequency activity appears briefly following a sensory stimulus, the spectrogram will display a short burst of energy at the corresponding frequencies.

Unlike a conventional frequency spectrum, the spectrogram immediately reveals when these changes occur.

This ability to visualise temporal changes makes spectrograms especially valuable for analysing nonstationary neural signals.

### Figure 6.5

EEG Waveform ↔ Corresponding Spectrogram

(Insert a figure showing an EEG waveform alongside its spectrogram, highlighting how changes in the waveform correspond to changes in the time–frequency representation.)

# 6.3.7 Applications in Neural Signal Processing

Spectrograms are widely used because many neural signals vary continuously over time.

Some common applications include:

### EEG Analysis

Visualising changes in neural oscillations during cognitive tasks, sleep, or sensory stimulation.

### EMG Analysis

Observing how muscle activation changes during movement or fatigue.

### Brain–Computer Interfaces

Monitoring time-varying neural rhythms used for feature extraction and classification.

### Event-Related Studies

Examining changes in neural activity before, during, and after stimulus presentation.

# 6.3.8 Advantages

Spectrograms provide several important advantages.

Simultaneously display time and frequency information.

Make nonstationary signals easier to interpret.

Provide an intuitive visual representation of STFT results.

Help identify transient neural events.

# 6.3.9 Limitations

Although spectrograms are powerful visualisation tools, they inherit the limitations of the STFT.

The quality of a spectrogram depends on the chosen window size.

A small window improves time resolution but reduces frequency resolution.

A large window improves frequency resolution but reduces time resolution.

Therefore, every spectrogram reflects the same time–frequency trade-off introduced by the Short-Time Fourier Transform.

### Important Note

A spectrogram is not a new analysis technique.

It is simply a visual representation of the Short-Time Fourier Transform.

Understanding this relationship is important because improvements in the underlying STFT directly affect the appearance of the spectrogram.

# 6.3.10 Summary

In this section, we introduced the spectrogram, one of the most widely used visualisation tools in time–frequency analysis.

We learned that a spectrogram is generated from the Short-Time Fourier Transform and provides a graphical representation of how the frequency content of a signal changes over time.

By displaying time, frequency, and signal intensity in a single figure, spectrograms allow us to visualise nonstationary neural signals much more effectively than a conventional frequency spectrum.

## Transition to Section 6.4

The spectrogram represents a major improvement over conventional frequency analysis because it shows both when and at what frequency neural activity occurs.

However, it still depends on the fixed analysis window used by the Short-Time Fourier Transform.

This fixed window creates a compromise between time resolution and frequency resolution.

Is there a method that can adapt its analysis to different types of neural activity?

The answer is wavelet analysis, which we will introduce in the next section.