# 6.4 Introduction to Wavelet Analysis

Duration: ~8 Minutes

# 6.4.1 Introduction

In the previous sections, we learned that the Short-Time Fourier Transform allows us to analyse both the time and frequency content of a neural signal. We also learned that the resulting spectrogram provides a visual representation of how frequency components change over time.

Although the Short-Time Fourier Transform represents a significant improvement over the conventional Fourier Transform, it still has an important limitation.

It uses a fixed analysis window throughout the entire signal.

This means that the same window is used whether the signal changes slowly over several seconds or rapidly within a few milliseconds.

Unfortunately, neural signals often contain both slow oscillations and fast transient events.

A single fixed window cannot analyse both equally well.

To overcome this limitation, a more flexible approach called wavelet analysis was developed.

# 6.4.2 Learning Objectives

After completing this section, you should be able to:

Explain why wavelet analysis was developed.

Describe the basic principle of wavelet analysis.

Understand the concept of multi-resolution analysis.

Compare wavelet analysis with the Short-Time Fourier Transform.

Recognise common applications of wavelet analysis in neuroscience.

# 6.4.3 Why Do We Need Wavelet Analysis?

### Lecture

Let us first revisit the limitation of the Short-Time Fourier Transform.

Suppose we choose a large analysis window.

The large window provides excellent frequency resolution because it contains many data points.

However, if a brief neural event occurs, the exact timing of that event becomes difficult to determine.

Now suppose we choose a small analysis window.

The timing of rapid events can now be identified accurately.

However, because the window contains fewer data points, frequency resolution becomes poorer.

This trade-off exists because the window size remains fixed throughout the analysis.

Neural signals rarely behave this way.

Some neural events develop slowly over time, while others occur almost instantaneously.

Ideally, we would like the analysis window to adapt automatically to the characteristics of the signal.

This is the fundamental idea behind wavelet analysis.

### Important Note

STFT uses one fixed window.

Wavelet analysis uses windows of different sizes depending on the signal being analysed.

# 6.4.4 What is Wavelet Analysis?

### Definition

Wavelet analysis is a time–frequency analysis technique that analyses a signal using wavelets of different scales, allowing different time and frequency resolutions for different parts of the signal.

### Lecture

Unlike the Fourier Transform, which represents a signal using sine and cosine waves, wavelet analysis represents a signal using short wave-like functions called wavelets.

These wavelets can be stretched or compressed depending on the part of the signal being analysed.

For slowly varying components, the wavelet becomes wider, providing better frequency resolution.

For rapidly changing events, the wavelet becomes narrower, providing better time resolution.

Because the analysing function changes its size, wavelet analysis can adapt more effectively to nonstationary neural signals.

### Key Definition

Wavelet: A short-duration oscillatory function used to analyse signals at different scales.

# 6.4.5 Multi-Resolution Analysis

One of the most important concepts in wavelet analysis is multi-resolution analysis.

### Lecture

Instead of examining the entire signal using one fixed window, wavelet analysis examines the signal at multiple scales.

Large wavelets are used to analyse slower components of the signal.

Small wavelets are used to analyse rapid transient events.

As a result, different parts of the same signal are analysed with different resolutions.

This adaptive behaviour makes wavelet analysis particularly suitable for neural recordings, where both slow oscillations and brief events often occur within the same recording.

(Insert Figure 6.6: Comparison of STFT using a fixed window and wavelet analysis using multiple window sizes.)

# 6.4.6 STFT versus Wavelet Analysis

The differences between the two techniques can be summarised as follows.

Rather than replacing the STFT, wavelet analysis extends the range of signals that can be analysed effectively.

Both techniques remain valuable, and the choice depends on the characteristics of the data and the objectives of the study.

# 6.4.7 Applications in Neural Signal Processing

Wavelet analysis has become an important tool in modern neuroscience because many neural signals are highly nonstationary.

Some common applications include:

### EEG Analysis

Detecting transient oscillatory activity and analysing brain rhythms that change rapidly over time.

### Event-Related Brain Activity

Studying neural responses immediately before and after sensory or cognitive events.

### Epilepsy Research

Identifying short-duration epileptic discharges that may be difficult to detect using conventional frequency analysis.

### Brain–Computer Interfaces

Extracting time-varying features from EEG signals for classification and control applications.

# 6.4.8 Advantages

Wavelet analysis offers several important advantages.

Adapts to signals that change over time.

Provides good time resolution for rapid events.

Provides good frequency resolution for slower oscillations.

Well suited for analysing nonstationary neural signals.

# 6.4.9 Limitations

Although wavelet analysis is highly flexible, it is not without limitations.

The results depend on the choice of the mother wavelet, and different wavelets may be better suited for different applications.

In addition, wavelet analysis is generally more computationally demanding than the Short-Time Fourier Transform.

Therefore, selecting an appropriate wavelet requires an understanding of both the signal and the objectives of the analysis.

### Important Note

Wavelet analysis is not simply a better version of the STFT.

Both techniques are valuable.

The choice depends on the nature of the neural signal and the scientific question being investigated.

# 6.4.10 Summary

In this section, we introduced wavelet analysis, an adaptive time–frequency analysis technique designed to overcome the fixed-window limitation of the Short-Time Fourier Transform.

We learned that wavelet analysis examines signals at multiple scales, providing good time resolution for rapid events and good frequency resolution for slower oscillations.

This adaptability makes wavelet analysis particularly useful for analysing highly nonstationary neural signals, such as EEG recordings containing transient neural events.

## Transition to Section 6.5

So far, we have focused on methods for analysing how neural activity changes over time and across different frequencies.

However, many neuroscience experiments are designed around specific events, such as the presentation of a visual image, an auditory tone, or a button press.

Rather than analysing continuous neural activity, researchers often wish to measure the brain's response to these events.

One of the most widely used approaches for this purpose is the Event-Related Potential (ERP), which we will explore in the next section.