# 6.5 Event-Related Potentials (ERPs)

Duration: ~7 Minutes

# 6.5.1 Introduction

In the previous sections, we studied techniques such as the Short-Time Fourier Transform, spectrograms, and wavelet analysis to understand how neural activity changes over time and across different frequency ranges.

While these techniques analyse continuous neural activity, many neuroscience experiments are designed around specific events.

For example, a participant may hear a sound, see an image, receive a tactile stimulus, or press a button in response to a task.

Researchers are often interested in understanding how the brain responds immediately after such events.

Rather than analysing the entire EEG recording, they focus on the neural activity that occurs around each event.

One of the most widely used methods for studying these responses is the Event-Related Potential, commonly known as the ERP.

# 6.5.2 Learning Objectives

After completing this section, you should be able to:

Define an Event-Related Potential (ERP).

Explain how ERPs are obtained from EEG recordings.

Describe the process of stimulus averaging.

Understand why averaging improves ERP detection.

Recognise common applications of ERP analysis in neuroscience.

# 6.5.3 What is an Event-Related Potential?

### Definition

An Event-Related Potential (ERP) is the electrical response of the brain that is time-locked to a specific sensory, cognitive, or motor event.

### Lecture

Unlike continuous EEG analysis, ERP analysis focuses on neural activity occurring immediately before and after a particular event.

The event may be:

The presentation of a visual image.

An auditory tone.

A tactile stimulus.

A movement initiated by the participant.

A cognitive decision.

The objective is to determine how the brain responds to that specific event.

Instead of analysing the entire recording, researchers repeatedly examine short EEG segments surrounding each occurrence of the event.

### Key Definition

Event-Related Potential (ERP): The averaged electrical response of the brain that is time-locked to a specific event.

# 6.5.4 Why is Averaging Necessary?

### Lecture

A single EEG recording contains a mixture of many different neural processes.

The brain is continuously active, even when no experimental task is being performed.

As a result, the response to a single stimulus is often very small compared with the ongoing background EEG.

If we examine only one trial, the event-related response may be difficult to distinguish from the surrounding neural activity.

To improve the visibility of the response, the same stimulus is presented repeatedly.

For each presentation, a short EEG segment is extracted around the event.

These segments are then averaged together.

Because the event-related response occurs at approximately the same time in every trial, it is reinforced during averaging.

In contrast, much of the unrelated background EEG varies randomly from trial to trial and therefore tends to cancel out.

The result is a much clearer representation of the brain's response to the stimulus.

### Figure 6.7

Repeated Stimuli → Individual EEG Epochs → Averaging → ERP Waveform

(Insert a figure showing multiple EEG epochs aligned to stimulus onset and the resulting averaged ERP.)

# 6.5.5 ERP Components

An ERP waveform consists of a series of positive and negative voltage deflections.

These deflections are referred to as ERP components.

They are usually named according to:

Their polarity (P for positive, N for negative).

Their approximate latency after the stimulus.

For example:

P100 – positive peak occurring approximately 100 milliseconds after stimulus onset.

N100 – negative peak occurring approximately 100 milliseconds after stimulus onset.

P300 – positive peak occurring approximately 300 milliseconds after stimulus onset.

Different ERP components are associated with different stages of sensory, perceptual, or cognitive processing.

(Insert Figure 6.8: Typical ERP waveform showing labelled components such as P100, N100, and P300.)

# 6.5.6 Applications of ERP Analysis

ERP analysis is widely used because it provides excellent temporal resolution, allowing researchers to study how rapidly the brain responds to events.

Some common applications include:

### Sensory Processing

Studying the brain's response to visual, auditory, or tactile stimuli.

### Cognitive Neuroscience

Investigating attention, perception, memory, language, and decision-making.

### Clinical Research

Examining changes in neural processing associated with neurological or psychiatric disorders.

### Brain–Computer Interfaces

Detecting stimulus-related brain responses, such as the P300, for communication and control applications.

# 6.5.7 Advantages

ERP analysis provides several important advantages.

Excellent temporal resolution.

Direct measurement of stimulus-related neural activity.

Widely used in cognitive neuroscience.

Relatively simple experimental design.

# 6.5.8 Limitations

Although ERP analysis is a powerful technique, it also has several limitations.

The most important limitation is that the event-related response is often much smaller than the ongoing EEG activity.

For this reason, many repeated trials are usually required to obtain a reliable ERP waveform.

In addition, ERP analysis assumes that the neural response occurs at approximately the same time following each presentation of the stimulus.

If the timing of the response varies substantially between trials, averaging may reduce the visibility of the ERP.

### Important Note

An ERP is not obtained from a single EEG recording.

It is typically obtained by averaging many EEG epochs that are aligned to the same experimental event.

This averaging process is the key reason why ERPs can reveal neural responses that are difficult to observe in individual trials.

# 6.5.9 Summary

In this section, we introduced Event-Related Potentials (ERPs) as a method for studying the brain's response to specific events.

We learned that ERP analysis focuses on short EEG segments surrounding repeated stimulus presentations and uses averaging to enhance the event-related response while reducing unrelated background activity.

Because ERPs provide precise information about the timing of neural processing, they are widely used in neuroscience, psychology, clinical research, and brain–computer interface applications.

## Transition to Section 6.6

Throughout this section, we assumed that the brain's response occurs at approximately the same time after every stimulus.

However, not all neural activity behaves in this way.

Some neural responses occur at a consistent time relative to the stimulus, whereas others involve changes in oscillatory activity that are related to the event but are not precisely time-locked.

Understanding the difference between these two types of neural responses is essential for interpreting EEG data.

In the final section of this module, we will distinguish between time-locked neural activity and induced neural activity.