# Module 6: Nonstationary Neural Signals

Duration: 45 Minutes

# Module Overview

In the previous module, we introduced signal filtering and learned how different filters improve the quality of neural recordings by removing unwanted signal components. We also explored the frequency-domain representation of neural signals and saw that frequency analysis provides valuable information about the oscillatory behaviour of neural activity.

However, an important assumption has been made throughout much of our discussion so far.

We have implicitly assumed that the frequency content of the signal remains relatively constant throughout the recording.

In many engineering applications, this assumption is reasonable. Unfortunately, it is rarely true for neural signals.

Brain activity changes continuously. Neural oscillations appear and disappear, muscle activity begins and ends, responses to external stimuli occur over very short time intervals, and cognitive processes evolve dynamically over time.

As a result, neural signals are generally nonstationary, meaning that their statistical and spectral characteristics change over time.

To analyse such signals, we require techniques that describe both when and at what frequency particular neural events occur.

In this module, we will introduce the concept of nonstationary neural signals and study the principal techniques used to analyse them, including the Short-Time Fourier Transform (STFT), spectrograms, wavelet analysis, event-related potentials (ERPs), and the distinction between time-locked and induced neural activity.

By the end of this module, you will understand why conventional frequency analysis is often insufficient for neural signals and how modern time-frequency methods overcome these limitations.

# Learning Outcomes

After completing this module, you should be able to:

- Explain why neural signals are considered nonstationary.

- Describe the limitations of conventional Fourier analysis for time-varying signals.

- Explain the principle of the Short-Time Fourier Transform.

- Interpret a spectrogram.

- Describe the basic idea behind wavelet analysis.

- Explain the concept of event-related potentials.

- Distinguish between time-locked and induced neural activity.

# Module Structure

| Section | Topic | Duration |
| --- | --- | --- |
| 6.1 | Why Neural Signals Change with Time | 8 min |
| 6.2 | Short-Time Fourier Transform | 8 min |
| 6.3 | Spectrograms | 8 min |
| 6.4 | Introduction to Wavelet Analysis | 8 min |
| 6.5 | Event-Related Potentials | 7 min |
| 6.6 | Time-Locked and Induced Neural Activity | 6 min |

# 6.1 Why Neural Signals Change with Time

Duration: ~8 Minutes

# 6.1.1 Introduction

In the previous module, we learned how filtering improves the quality of neural recordings by removing unwanted signal components. We also became familiar with the idea that neural signals can be analysed in the frequency domain to identify the oscillations present within a recording.

At this point, an important question naturally arises.

Do the frequencies present in a neural signal remain constant throughout the entire recording?

For many real-world signals, including neural recordings, the answer is no.

Neural activity is continuously changing. A person may transition from rest to movement, respond to an external stimulus, fall asleep, or perform a cognitive task. Each of these events changes the underlying neural activity, and consequently, the characteristics of the recorded signal also change over time.

Understanding these changes is the first step toward analysing nonstationary neural signals.

# 6.1.2 Learning Objectives

After completing this section, you should be able to:

- Define a stationary and a nonstationary signal.

- Explain why neural signals are generally nonstationary.

- Identify common causes of time-varying neural activity.

- Understand why time-varying signals require specialised analysis techniques.

# 6.1.3 What is a Stationary Signal?

### Definition

A stationary signal is a signal whose statistical and spectral characteristics remain essentially constant over time.

### Lecture

Imagine recording the continuous hum of an electrical generator operating under stable conditions.

If you observe the signal for one second, five seconds, or even one minute, its overall characteristics remain largely unchanged.

The average amplitude remains similar.

The frequency content remains similar.

The statistical properties remain relatively constant.

This type of signal is called stationary because its behaviour does not change significantly with time.

Many classical signal processing techniques were originally developed under this assumption.

### Key Definition

Stationary Signal: A signal whose statistical properties remain approximately constant throughout the recording.

# 6.1.4 What is a Nonstationary Signal?

### Definition

A nonstationary signal is a signal whose statistical or spectral characteristics change over time.

### Lecture

Now consider an EEG recording obtained while a participant performs a cognitive task.

At the beginning of the recording, the participant may be relaxed with their eyes closed.

A few seconds later, they open their eyes.

Soon afterward, they respond to a visual stimulus.

Later, they perform a memory task.

Throughout this recording, neural activity changes continuously.

Some oscillations become stronger.

Others become weaker.

New frequency components may appear, while existing ones disappear.

The signal recorded at the beginning is therefore different from the signal recorded later.

This type of signal is called nonstationary because its characteristics evolve with time.

Unlike stationary signals, nonstationary signals cannot be fully described using a single frequency spectrum for the entire recording.

### Key Definition

Nonstationary Signal: A signal whose statistical or frequency characteristics change with time.

# 6.1.5 Why Are Neural Signals Nonstationary?

Neural signals reflect ongoing physiological activity within the nervous system.

Since brain activity continuously changes in response to both internal and external events, neural recordings are naturally nonstationary.

Several factors contribute to these temporal changes.

### Changes in Behaviour

Neural activity differs depending on whether a person is resting, walking, speaking, or performing a task.

As behaviour changes, the recorded signal also changes.

### Sensory Stimulation

Visual, auditory, or tactile stimuli produce transient neural responses.

These responses occur only for short periods of time and therefore introduce time-varying changes into the recording.

### Cognitive Processes

Attention, memory, decision-making, and learning involve dynamic patterns of neural activity.

These cognitive processes evolve continuously and influence the recorded signal.

### Physiological State

Changes in sleep stage, alertness, fatigue, or emotional state also modify neural activity over time.

Consequently, recordings obtained during different physiological states exhibit different signal characteristics.

# 6.1.6 An Example Using EEG

Consider a thirty-second EEG recording.

During the first ten seconds, the participant sits quietly with their eyes closed.

Alpha activity is prominent.

At ten seconds, the participant opens their eyes.

The alpha rhythm decreases.

At twenty seconds, a visual stimulus is presented.

A brief event-related response appears.

Although all three periods belong to the same recording, their frequency content is clearly different.

This demonstrates that neural activity evolves continuously rather than remaining constant throughout the recording.

(Insert Figure 6.1: EEG waveform showing changes in activity over time, with labels indicating eyes closed, eyes open, and stimulus presentation.)

# 6.1.7 Why Does This Create a Problem?

At first glance, analysing the frequency content of a neural signal seems straightforward.

However, most classical frequency analysis methods assume that the signal remains unchanged throughout the observation period.

If the frequency content changes during the recording, calculating a single frequency spectrum for the entire signal provides only an overall average.

It does not indicate when specific frequencies were present.

For example, suppose alpha activity is present only during the first ten seconds of an EEG recording.

A conventional Fourier Transform may indicate that alpha frequencies exist somewhere in the recording, but it cannot identify the exact time at which they occurred.

For many neuroscience applications, this information is essential.

We often need to know not only what frequencies are present, but also when they occur.

### Important Note

The question in neural signal analysis is no longer simply:

"Which frequencies are present?"

Instead, we ask:

"Which frequencies are present, and when do they occur?"

This question motivates the development of time–frequency analysis, which will be introduced in the next section.

# 6.1.8 Summary

In this section, we introduced the concept of nonstationary neural signals.

We learned that neural recordings are generally nonstationary because brain activity changes continuously in response to behaviour, sensory stimulation, cognitive processes, and physiological state.

Unlike stationary signals, nonstationary signals cannot be fully described using a single frequency spectrum because their frequency content changes over time.

Understanding this limitation prepares us for the next topic—the Short-Time Fourier Transform (STFT), which extends conventional frequency analysis by incorporating information about both time and frequency.

## Transition to Section 6.2

We now understand the problem.

Neural signals change continuously over time, but the conventional Fourier Transform provides only a single frequency spectrum for the entire recording.

So how can we determine which frequencies are present at different moments in time?

The solution is the Short-Time Fourier Transform (STFT), which we will study in the next section.

# 6.2 Short-Time Fourier Transform (STFT)

Duration: ~8 Minutes

# 6.2.1 Introduction

In the previous section, we learned that neural signals are generally nonstationary. Their frequency content changes continuously as neural activity evolves over time.

We also identified an important limitation of the conventional Fourier Transform.

Although it tells us which frequencies are present in a signal, it cannot tell us when those frequencies occur.

For many neuroscience applications, this information is essential.

Imagine analysing an EEG recording during a visual experiment.

Suppose alpha activity appears only after the participant closes their eyes.

The Fourier Transform can indicate that alpha frequencies exist somewhere in the recording, but it cannot identify the exact moment at which they appeared.

To solve this problem, we need a method that analyses both time and frequency simultaneously.

One of the earliest and most widely used solutions is the Short-Time Fourier Transform, commonly abbreviated as STFT.

# 6.2.2 Learning Objectives

After completing this section, you should be able to:

- Explain why the Short-Time Fourier Transform was developed.

- Describe the basic principle of STFT.

- Understand the concept of a moving analysis window.

- Explain how STFT provides both time and frequency information.

- Recognise the advantages and limitations of STFT.

# 6.2.3 Why Do We Need STFT?

### Lecture

Let us begin by revisiting the problem.

Suppose you record an EEG signal for thirty seconds.

During the first ten seconds, alpha activity is dominant.

During the next ten seconds, beta activity becomes stronger.

Finally, during the last ten seconds, gamma activity appears briefly.

If we apply a conventional Fourier Transform to the entire thirty-second recording, the result is a single frequency spectrum representing the entire signal.

Although this spectrum tells us that alpha, beta, and gamma frequencies are present, it provides no information about when each frequency occurred.

This is a significant limitation because neural activity is dynamic.

In neuroscience, timing is often just as important as frequency.

To understand brain function, we need a method that tells us both:

- Which frequencies are present?

- At what time do they occur?

This is precisely the problem that the Short-Time Fourier Transform was designed to solve.

### Important Note

The Fourier Transform answers "What frequencies exist?"

The Short-Time Fourier Transform answers "What frequencies exist, and when do they occur?"

# 6.2.4 What is the Short-Time Fourier Transform?

### Definition

The Short-Time Fourier Transform (STFT) is a time–frequency analysis technique that divides a signal into short time segments and performs a Fourier Transform on each segment separately.

### Lecture

Rather than analysing the entire signal at once, STFT analyses only a small portion of the signal at a time.

Imagine placing a small window over the beginning of the signal.

Only the data inside this window are analysed.

The window is then moved slightly forward, and the analysis is repeated.

This process continues until the entire signal has been examined.

Each window produces its own frequency spectrum.

When these spectra are combined, they reveal how the frequency content changes throughout the recording.

This simple idea transforms the Fourier Transform from a purely frequency-based analysis into a time-frequency analysis.

### Figure 6.2

Original Signal → Sliding Window → Fourier Transform for Each Window → Time-Frequency Representation

(Insert a diagram illustrating a sliding window moving across a signal, with a Fourier Transform applied to each window.)

# 6.2.5 How Does STFT Work?

The operation of STFT can be understood in four simple steps.

### Step 1 — Select a Short Time Window

A small portion of the signal is selected using a window function.

Only the data within this window are analysed.

### Step 2 — Apply the Fourier Transform

A Fourier Transform is performed on the data inside the window.

This provides the frequency content for that short time interval.

### Step 3 — Move the Window

The window is shifted slightly along the signal.

A new portion of the signal is selected.

### Step 4 — Repeat the Process

The Fourier Transform is computed again for the new window.

Repeating this process across the entire recording produces a sequence of frequency spectra describing how the signal changes over time.

# 6.2.6 Window Size and the Time–Frequency Trade-off

One of the most important concepts in STFT is the window size.

The size of the analysis window determines how the signal is examined.

### Small Window

A smaller window provides better time resolution because it can detect rapid changes in the signal.

However, it contains fewer data points, reducing frequency resolution.

### Large Window

A larger window contains more data points and therefore provides better frequency resolution.

However, it cannot localise rapid changes in time as accurately.

### Important Concept

This creates a fundamental trade-off.

A small window tells us when something happened more accurately.

A large window tells us which frequency is present more accurately.

Unfortunately, a single fixed window cannot optimise both simultaneously.

(Insert Figure 6.3: Comparison of small and large STFT windows showing the trade-off between time resolution and frequency resolution.)

### Important Note

STFT uses a fixed window size.

This means that the same window is used throughout the entire signal, regardless of whether the signal changes rapidly or slowly.

As we will see later, this fixed-window approach is one of the major limitations of STFT.

# 6.2.7 Applications in Neural Signal Processing

The Short-Time Fourier Transform is widely used because many neural signals change continuously over time.

Some common applications include:

### EEG

Tracking how neural oscillations change during cognitive tasks, sleep, or sensory stimulation.

### EMG

Observing changes in muscle activity during movement and fatigue.

### Event-Related Neural Activity

Examining frequency changes before, during, and after stimulus presentation.

### Brain–Computer Interfaces

Monitoring time-varying neural rhythms used for feature extraction and classification.

# 6.2.8 Advantages

The Short-Time Fourier Transform provides several important advantages.

- Describes both time and frequency.

- Suitable for analysing nonstationary signals.

- Relatively simple to implement.

- Forms the basis for spectrogram generation.

# 6.2.9 Limitations

Although STFT represents a major improvement over the conventional Fourier Transform, it is not a perfect solution.

Because it uses a single fixed window, it cannot simultaneously achieve high time resolution and high frequency resolution.

This limitation becomes particularly important when analysing neural signals that contain both slow oscillations and rapid transient events.

For this reason, more advanced techniques, such as wavelet analysis, were later developed.

# 6.2.10 Summary

In this section, we introduced the Short-Time Fourier Transform (STFT).

We learned that STFT overcomes one of the major limitations of the conventional Fourier Transform by analysing short segments of the signal instead of the entire recording.

This allows us to determine both the frequency content and its variation over time.

However, we also learned that STFT uses a fixed analysis window, creating a trade-off between time resolution and frequency resolution.

## Transition to Section 6.3

The output of the Short-Time Fourier Transform consists of a sequence of frequency spectra, one for each time window.

While this information is valuable, examining hundreds of separate spectra is neither practical nor intuitive.

Instead, we need a way to visualise how frequency changes over time in a single image.

This visual representation is known as a spectrogram, and it will be the focus of the next section.

# 6.3 Spectrograms

Duration: ~8 Minutes

# 6.3.1 Introduction

In the previous section, we introduced the Short-Time Fourier Transform (STFT) and learned how it analyses a signal using a sequence of short, overlapping time windows. Unlike the conventional Fourier Transform, STFT provides information about both time and frequency, allowing us to study signals whose characteristics change over time.

However, performing an STFT produces a large number of individual frequency spectra—one for each analysis window.

Interpreting hundreds or even thousands of separate spectra is neither practical nor intuitive.

To solve this problem, the results of the STFT are displayed as a single visual representation called a spectrogram.

A spectrogram allows us to observe how the frequency content of a signal evolves over time, making it one of the most widely used visualization tools in neural signal processing.

# 6.3.2 Learning Objectives

After completing this section, you should be able to:

- Define a spectrogram.

- Explain how a spectrogram is generated.

- Interpret the different components of a spectrogram.

- Relate a spectrogram to the original neural signal.

- Describe common applications of spectrograms in neuroscience.

# 6.3.3 What is a Spectrogram?

### Definition

A spectrogram is a graphical representation of how the frequency content of a signal changes over time.

It is generated by computing the Short-Time Fourier Transform over consecutive segments of the signal.

### Lecture

Instead of displaying only one frequency spectrum, a spectrogram combines the spectra from all STFT windows into a single image.

This allows us to observe both when specific frequencies occur and how their strengths change over time.

A spectrogram transforms a complex sequence of frequency spectra into a visual map that is much easier to interpret.

For this reason, spectrograms are widely used in speech processing, biomedical engineering, and neuroscience.

### Key Definition

Spectrogram: A time–frequency representation showing how the frequency content of a signal changes over time.

# 6.3.4 Understanding a Spectrogram

A spectrogram consists of three main components.

### Time Axis

The horizontal axis represents time.

As we move from left to right, we observe how the signal evolves throughout the recording.

### Frequency Axis

The vertical axis represents frequency, usually measured in Hertz (Hz).

Lower frequencies appear near the bottom of the spectrogram, while higher frequencies appear near the top.

### Colour Intensity

The colour represents the strength or energy of each frequency component at a particular time.

Brighter or warmer colours generally indicate stronger activity.

Darker or cooler colours indicate weaker activity.

(Insert Figure 6.4: Example spectrogram with labelled time axis, frequency axis, and colour scale.)

# 6.3.5 How is a Spectrogram Generated?

A spectrogram is generated directly from the STFT.

The process consists of four steps.

### Step 1 — Divide the Signal into Short Windows

The signal is divided into a series of overlapping time segments.

### Step 2 — Compute the STFT

A Fourier Transform is performed for each window.

Each window produces one frequency spectrum.

### Step 3 — Arrange the Spectra in Time Order

The spectra are placed side by side according to the position of each analysis window.

### Step 4 — Display the Signal as an Image

The frequency spectra are represented using colour intensity, producing a two-dimensional image that simultaneously displays time, frequency, and signal strength.

# 6.3.6 Interpreting a Spectrogram

Learning to interpret a spectrogram is an important skill in neural signal processing.

Suppose we analyse an EEG recording.

If alpha activity appears only between 5 seconds and 10 seconds, the spectrogram will show increased energy around the alpha frequency range only during that time interval.

Similarly, if higher-frequency activity appears briefly following a sensory stimulus, the spectrogram will display a short burst of energy at the corresponding frequencies.

Unlike a conventional frequency spectrum, the spectrogram immediately reveals when these changes occur.

This ability to visualise temporal changes makes spectrograms especially valuable for analysing nonstationary neural signals.

### Figure 6.5

EEG Waveform ↔ Corresponding Spectrogram

(Insert a figure showing an EEG waveform alongside its spectrogram, highlighting how changes in the waveform correspond to changes in the time–frequency representation.)

# 6.3.7 Applications in Neural Signal Processing

Spectrograms are widely used because many neural signals vary continuously over time.

Some common applications include:

### EEG Analysis

Visualising changes in neural oscillations during cognitive tasks, sleep, or sensory stimulation.

### EMG Analysis

Observing how muscle activation changes during movement or fatigue.

### Brain–Computer Interfaces

Monitoring time-varying neural rhythms used for feature extraction and classification.

### Event-Related Studies

Examining changes in neural activity before, during, and after stimulus presentation.

# 6.3.8 Advantages

Spectrograms provide several important advantages.

- Simultaneously display time and frequency information.

- Make nonstationary signals easier to interpret.

- Provide an intuitive visual representation of STFT results.

- Help identify transient neural events.

# 6.3.9 Limitations

Although spectrograms are powerful visualisation tools, they inherit the limitations of the STFT.

The quality of a spectrogram depends on the chosen window size.

A small window improves time resolution but reduces frequency resolution.

A large window improves frequency resolution but reduces time resolution.

Therefore, every spectrogram reflects the same time–frequency trade-off introduced by the Short-Time Fourier Transform.

### Important Note

A spectrogram is not a new analysis technique.

It is simply a visual representation of the Short-Time Fourier Transform.

Understanding this relationship is important because improvements in the underlying STFT directly affect the appearance of the spectrogram.

# 6.3.10 Summary

In this section, we introduced the spectrogram, one of the most widely used visualisation tools in time–frequency analysis.

We learned that a spectrogram is generated from the Short-Time Fourier Transform and provides a graphical representation of how the frequency content of a signal changes over time.

By displaying time, frequency, and signal intensity in a single figure, spectrograms allow us to visualise nonstationary neural signals much more effectively than a conventional frequency spectrum.

## Transition to Section 6.4

The spectrogram represents a major improvement over conventional frequency analysis because it shows both when and at what frequency neural activity occurs.

However, it still depends on the fixed analysis window used by the Short-Time Fourier Transform.

This fixed window creates a compromise between time resolution and frequency resolution.

Is there a method that can adapt its analysis to different types of neural activity?

The answer is wavelet analysis, which we will introduce in the next section.

# 6.4 Introduction to Wavelet Analysis

Duration: ~8 Minutes

# 6.4.1 Introduction

In the previous sections, we learned that the Short-Time Fourier Transform allows us to analyse both the time and frequency content of a neural signal. We also learned that the resulting spectrogram provides a visual representation of how frequency components change over time.

Although the Short-Time Fourier Transform represents a significant improvement over the conventional Fourier Transform, it still has an important limitation.

It uses a fixed analysis window throughout the entire signal.

This means that the same window is used whether the signal changes slowly over several seconds or rapidly within a few milliseconds.

Unfortunately, neural signals often contain both slow oscillations and fast transient events.

A single fixed window cannot analyse both equally well.

To overcome this limitation, a more flexible approach called wavelet analysis was developed.

# 6.4.2 Learning Objectives

After completing this section, you should be able to:

- Explain why wavelet analysis was developed.

- Describe the basic principle of wavelet analysis.

- Understand the concept of multi-resolution analysis.

- Compare wavelet analysis with the Short-Time Fourier Transform.

- Recognise common applications of wavelet analysis in neuroscience.

# 6.4.3 Why Do We Need Wavelet Analysis?

### Lecture

Let us first revisit the limitation of the Short-Time Fourier Transform.

Suppose we choose a large analysis window.

The large window provides excellent frequency resolution because it contains many data points.

However, if a brief neural event occurs, the exact timing of that event becomes difficult to determine.

Now suppose we choose a small analysis window.

The timing of rapid events can now be identified accurately.

However, because the window contains fewer data points, frequency resolution becomes poorer.

This trade-off exists because the window size remains fixed throughout the analysis.

Neural signals rarely behave this way.

Some neural events develop slowly over time, while others occur almost instantaneously.

Ideally, we would like the analysis window to adapt automatically to the characteristics of the signal.

This is the fundamental idea behind wavelet analysis.

### Important Note

STFT uses one fixed window.

Wavelet analysis uses windows of different sizes depending on the signal being analysed.

# 6.4.4 What is Wavelet Analysis?

### Definition

Wavelet analysis is a time–frequency analysis technique that analyses a signal using wavelets of different scales, allowing different time and frequency resolutions for different parts of the signal.

### Lecture

Unlike the Fourier Transform, which represents a signal using sine and cosine waves, wavelet analysis represents a signal using short wave-like functions called wavelets.

These wavelets can be stretched or compressed depending on the part of the signal being analysed.

For slowly varying components, the wavelet becomes wider, providing better frequency resolution.

For rapidly changing events, the wavelet becomes narrower, providing better time resolution.

Because the analysing function changes its size, wavelet analysis can adapt more effectively to nonstationary neural signals.

### Key Definition

Wavelet: A short-duration oscillatory function used to analyse signals at different scales.

# 6.4.5 Multi-Resolution Analysis

One of the most important concepts in wavelet analysis is multi-resolution analysis.

### Lecture

Instead of examining the entire signal using one fixed window, wavelet analysis examines the signal at multiple scales.

Large wavelets are used to analyse slower components of the signal.

Small wavelets are used to analyse rapid transient events.

As a result, different parts of the same signal are analysed with different resolutions.

This adaptive behaviour makes wavelet analysis particularly suitable for neural recordings, where both slow oscillations and brief events often occur within the same recording.

(Insert Figure 6.6: Comparison of STFT using a fixed window and wavelet analysis using multiple window sizes.)

# 6.4.6 STFT versus Wavelet Analysis

The differences between the two techniques can be summarised as follows.

| Feature | STFT | Wavelet Analysis |
| --- | --- | --- |
| Analysis Window | Fixed | Variable |
| Time Resolution | Fixed | Adaptive |
| Frequency Resolution | Fixed | Adaptive |
| Suitable for | Moderately changing signals | Highly nonstationary signals |
| Flexibility | Lower | Higher |

Rather than replacing the STFT, wavelet analysis extends the range of signals that can be analysed effectively.

Both techniques remain valuable, and the choice depends on the characteristics of the data and the objectives of the study.

# 6.4.7 Applications in Neural Signal Processing

Wavelet analysis has become an important tool in modern neuroscience because many neural signals are highly nonstationary.

Some common applications include:

### EEG Analysis

Detecting transient oscillatory activity and analysing brain rhythms that change rapidly over time.

### Event-Related Brain Activity

Studying neural responses immediately before and after sensory or cognitive events.

### Epilepsy Research

Identifying short-duration epileptic discharges that may be difficult to detect using conventional frequency analysis.

### Brain–Computer Interfaces

Extracting time-varying features from EEG signals for classification and control applications.

# 6.4.8 Advantages

Wavelet analysis offers several important advantages.

- Adapts to signals that change over time.

- Provides good time resolution for rapid events.

- Provides good frequency resolution for slower oscillations.

- Well suited for analysing nonstationary neural signals.

# 6.4.9 Limitations

Although wavelet analysis is highly flexible, it is not without limitations.

The results depend on the choice of the mother wavelet, and different wavelets may be better suited for different applications.

In addition, wavelet analysis is generally more computationally demanding than the Short-Time Fourier Transform.

Therefore, selecting an appropriate wavelet requires an understanding of both the signal and the objectives of the analysis.

### Important Note

Wavelet analysis is not simply a better version of the STFT.

Both techniques are valuable.

The choice depends on the nature of the neural signal and the scientific question being investigated.

# 6.4.10 Summary

In this section, we introduced wavelet analysis, an adaptive time–frequency analysis technique designed to overcome the fixed-window limitation of the Short-Time Fourier Transform.

We learned that wavelet analysis examines signals at multiple scales, providing good time resolution for rapid events and good frequency resolution for slower oscillations.

This adaptability makes wavelet analysis particularly useful for analysing highly nonstationary neural signals, such as EEG recordings containing transient neural events.

## Transition to Section 6.5

So far, we have focused on methods for analysing how neural activity changes over time and across different frequencies.

However, many neuroscience experiments are designed around specific events, such as the presentation of a visual image, an auditory tone, or a button press.

Rather than analysing continuous neural activity, researchers often wish to measure the brain's response to these events.

One of the most widely used approaches for this purpose is the Event-Related Potential (ERP), which we will explore in the next section.

# 6.5 Event-Related Potentials (ERPs)

Duration: ~7 Minutes

# 6.5.1 Introduction

In the previous sections, we studied techniques such as the Short-Time Fourier Transform, spectrograms, and wavelet analysis to understand how neural activity changes over time and across different frequency ranges.

While these techniques analyse continuous neural activity, many neuroscience experiments are designed around specific events.

For example, a participant may hear a sound, see an image, receive a tactile stimulus, or press a button in response to a task.

Researchers are often interested in understanding how the brain responds immediately after such events.

Rather than analysing the entire EEG recording, they focus on the neural activity that occurs around each event.

One of the most widely used methods for studying these responses is the Event-Related Potential, commonly known as the ERP.

# 6.5.2 Learning Objectives

After completing this section, you should be able to:

- Define an Event-Related Potential (ERP).

- Explain how ERPs are obtained from EEG recordings.

- Describe the process of stimulus averaging.

- Understand why averaging improves ERP detection.

- Recognise common applications of ERP analysis in neuroscience.

# 6.5.3 What is an Event-Related Potential?

### Definition

An Event-Related Potential (ERP) is the electrical response of the brain that is time-locked to a specific sensory, cognitive, or motor event.

### Lecture

Unlike continuous EEG analysis, ERP analysis focuses on neural activity occurring immediately before and after a particular event.

The event may be:

- The presentation of a visual image.

- An auditory tone.

- A tactile stimulus.

- A movement initiated by the participant.

- A cognitive decision.

The objective is to determine how the brain responds to that specific event.

Instead of analysing the entire recording, researchers repeatedly examine short EEG segments surrounding each occurrence of the event.

### Key Definition

Event-Related Potential (ERP): The averaged electrical response of the brain that is time-locked to a specific event.

# 6.5.4 Why is Averaging Necessary?

### Lecture

A single EEG recording contains a mixture of many different neural processes.

The brain is continuously active, even when no experimental task is being performed.

As a result, the response to a single stimulus is often very small compared with the ongoing background EEG.

If we examine only one trial, the event-related response may be difficult to distinguish from the surrounding neural activity.

To improve the visibility of the response, the same stimulus is presented repeatedly.

For each presentation, a short EEG segment is extracted around the event.

These segments are then averaged together.

Because the event-related response occurs at approximately the same time in every trial, it is reinforced during averaging.

In contrast, much of the unrelated background EEG varies randomly from trial to trial and therefore tends to cancel out.

The result is a much clearer representation of the brain's response to the stimulus.

### Figure 6.7

Repeated Stimuli → Individual EEG Epochs → Averaging → ERP Waveform

(Insert a figure showing multiple EEG epochs aligned to stimulus onset and the resulting averaged ERP.)

# 6.5.5 ERP Components

An ERP waveform consists of a series of positive and negative voltage deflections.

These deflections are referred to as ERP components.

They are usually named according to:

- Their polarity (P for positive, N for negative).

- Their approximate latency after the stimulus.

For example:

- P100 – positive peak occurring approximately 100 milliseconds after stimulus onset.

- N100 – negative peak occurring approximately 100 milliseconds after stimulus onset.

- P300 – positive peak occurring approximately 300 milliseconds after stimulus onset.

Different ERP components are associated with different stages of sensory, perceptual, or cognitive processing.

(Insert Figure 6.8: Typical ERP waveform showing labelled components such as P100, N100, and P300.)

# 6.5.6 Applications of ERP Analysis

ERP analysis is widely used because it provides excellent temporal resolution, allowing researchers to study how rapidly the brain responds to events.

Some common applications include:

### Sensory Processing

Studying the brain's response to visual, auditory, or tactile stimuli.

### Cognitive Neuroscience

Investigating attention, perception, memory, language, and decision-making.

### Clinical Research

Examining changes in neural processing associated with neurological or psychiatric disorders.

### Brain–Computer Interfaces

Detecting stimulus-related brain responses, such as the P300, for communication and control applications.

# 6.5.7 Advantages

ERP analysis provides several important advantages.

- Excellent temporal resolution.

- Direct measurement of stimulus-related neural activity.

- Widely used in cognitive neuroscience.

- Relatively simple experimental design.

# 6.5.8 Limitations

Although ERP analysis is a powerful technique, it also has several limitations.

The most important limitation is that the event-related response is often much smaller than the ongoing EEG activity.

For this reason, many repeated trials are usually required to obtain a reliable ERP waveform.

In addition, ERP analysis assumes that the neural response occurs at approximately the same time following each presentation of the stimulus.

If the timing of the response varies substantially between trials, averaging may reduce the visibility of the ERP.

### Important Note

An ERP is not obtained from a single EEG recording.

It is typically obtained by averaging many EEG epochs that are aligned to the same experimental event.

This averaging process is the key reason why ERPs can reveal neural responses that are difficult to observe in individual trials.

# 6.5.9 Summary

In this section, we introduced Event-Related Potentials (ERPs) as a method for studying the brain's response to specific events.

We learned that ERP analysis focuses on short EEG segments surrounding repeated stimulus presentations and uses averaging to enhance the event-related response while reducing unrelated background activity.

Because ERPs provide precise information about the timing of neural processing, they are widely used in neuroscience, psychology, clinical research, and brain–computer interface applications.

## Transition to Section 6.6

Throughout this section, we assumed that the brain's response occurs at approximately the same time after every stimulus.

However, not all neural activity behaves in this way.

Some neural responses occur at a consistent time relative to the stimulus, whereas others involve changes in oscillatory activity that are related to the event but are not precisely time-locked.

Understanding the difference between these two types of neural responses is essential for interpreting EEG data.

In the final section of this module, we will distinguish between time-locked neural activity and induced neural activity.

# 6.6 Time-Locked and Induced Neural Activity

Duration: ~6 Minutes

# 6.6.1 Introduction

In the previous section, we learned that Event-Related Potentials are obtained by averaging EEG responses that are aligned to repeated presentations of the same stimulus.

This averaging process works because certain neural responses occur at approximately the same time following every stimulus presentation.

However, not every neural response behaves in this way.

Some neural activities occur consistently at a fixed time after the event, while others vary in their timing from trial to trial. Although these responses are still related to the event, they are not precisely synchronized across repeated trials.

This distinction gives rise to two important concepts in neural signal analysis:

- Time-Locked Neural Activity

- Induced Neural Activity

Understanding the difference between these two types of responses is essential for selecting appropriate analysis methods and correctly interpreting neural recordings.

# 6.6.2 Learning Objectives

After completing this section, you should be able to:

- Define time-locked neural activity.

- Define induced neural activity.

- Distinguish between these two types of neural responses.

- Explain why ERPs mainly reflect time-locked activity.

- Recognise when time–frequency analysis is needed.

# 6.6.3 Time-Locked Neural Activity

### Definition

Time-locked neural activity refers to brain responses that occur at approximately the same time relative to each presentation of a stimulus.

### Lecture

Suppose a participant hears the same tone repeatedly.

Each time the tone is presented, the brain generates a response approximately 100 milliseconds after the stimulus.

Although the exact amplitude of the response may vary slightly, its timing remains relatively consistent across trials.

Because these responses occur at nearly the same latency, they become stronger when multiple trials are averaged together.

This is precisely the type of neural activity that produces Event-Related Potentials.

### Example

Imagine presenting a visual flash 100 times.

If a neural response consistently appears about 100 milliseconds after each flash, averaging the EEG recordings will reinforce that response.

As more trials are averaged, the response becomes clearer because its timing remains consistent across repetitions.

(Insert Figure 6.9: Multiple EEG epochs aligned to stimulus onset showing consistent response timing and the resulting ERP after averaging.)

### Key Definition

Time-Locked Activity: Neural activity that occurs at a consistent time relative to a repeated event.

# 6.6.4 Induced Neural Activity

### Definition

Induced neural activity refers to event-related changes in brain activity that do not occur at exactly the same time across repeated trials.

### Lecture

Now consider a different situation.

Suppose a participant performs a mental imagery task.

The task consistently changes brain activity, but the exact timing of these changes varies slightly from one trial to another.

One response may begin after 250 milliseconds.

Another may begin after 320 milliseconds.

A third may begin after 280 milliseconds.

Although all of these responses are related to the same task, they are not perfectly synchronized in time.

If these trials are averaged in the same way as an ERP, much of the induced activity will cancel out because its timing differs across trials.

Therefore, induced neural activity is usually studied using time–frequency analysis, such as spectrograms or wavelet analysis, rather than simple averaging.

### Figure 6.10

Multiple EEG epochs showing variable response timing across trials.

(Illustrate that averaging reduces the visibility of induced activity because the responses are not perfectly aligned.)

# 6.6.5 Comparing Time-Locked and Induced Activity

Although both types of neural activity are associated with an event, they differ in an important way.

| Feature | Time-Locked Activity | Induced Activity |
| --- | --- | --- |
| Relationship to stimulus | Occurs at a consistent latency | Related to the stimulus but timing varies |
| Trial-to-trial timing | Relatively consistent | Variable |
| ERP averaging | Clearly visible after averaging | Often reduced or lost after averaging |
| Preferred analysis | Event-Related Potentials (ERP) | Time–frequency analysis (STFT, spectrograms, wavelets) |

The key difference is timing consistency.

Time-locked responses remain aligned across repeated trials, whereas induced responses vary in their timing.

# 6.6.6 Why is This Distinction Important?

Understanding the difference between these two response types helps researchers choose the appropriate analysis method.

If the objective is to measure neural responses that occur consistently after each stimulus, ERP analysis is often appropriate.

However, if the objective is to study changes in neural oscillations whose timing varies across trials, time–frequency analysis provides more suitable information.

This is one of the reasons why modern neuroscience frequently combines ERP analysis with techniques such as spectrograms and wavelet analysis.

Together, these methods provide complementary insights into neural activity.

### Important Note

ERP analysis primarily reveals time-locked neural responses.

Time–frequency methods are particularly useful for studying induced neural activity that may not be visible after averaging.

# 6.6.7 Summary

In this section, we distinguished between time-locked and induced neural activity.

We learned that time-locked responses occur at approximately the same latency after repeated events and therefore become clearer through ERP averaging.

In contrast, induced neural activity is related to the event but varies in timing across trials. Because averaging reduces these responses, they are typically investigated using time–frequency analysis methods such as the STFT, spectrograms, and wavelet analysis.

Understanding this distinction is essential for selecting appropriate analytical techniques and correctly interpreting neural recordings.

# Module 6 Recap

In this module, we explored the analysis of nonstationary neural signals and learned why conventional frequency analysis is often insufficient for studying neural recordings.

The key concepts covered in this module are:

- Neural signals are generally nonstationary because their characteristics change over time.

- The Short-Time Fourier Transform (STFT) extends Fourier analysis by providing both time and frequency information.

- A spectrogram visualizes the output of the STFT, showing how frequency content changes throughout a recording.

- Wavelet analysis overcomes the fixed-window limitation of the STFT by using multiple analysis scales.

- Event-Related Potentials (ERPs) measure neural responses that are time-locked to specific events through averaging across repeated trials.

- Neural responses can be classified as time-locked or induced, and each requires different analysis approaches.

Together, these concepts provide a strong foundation for analysing dynamic neural activity and prepare us for more advanced topics in neural signal processing and neuroscience applications.