# 2.3 Amplification and Signal Conditioning

Duration: ~10 Minutes

## 2.3.1 Introduction

In the previous section, we learned how electrodes and sensors detect biological signals. However, detecting a signal is only the first step.

The electrical signals generated by the human body are extremely small. For example, EEG signals are typically measured in microvolts, which are millions of times smaller than the voltages used by most electronic devices. If these tiny signals were sent directly to a computer, they would be difficult to distinguish from electrical noise and could not be processed reliably.

To overcome this problem, every neural recording system includes a stage known as amplification and signal conditioning.

In this section, we will understand why amplification is necessary, how amplifiers improve signal quality, and what is meant by signal conditioning.

## 2.3.2 Learning Objectives

After completing this section, you should be able to:

- Explain why neural signals require amplification.

- Describe the function of an amplifier.

- Understand the concept of signal conditioning.

- Explain why amplification must occur before further signal processing.

# 2.3.3 Why Do Neural Signals Need Amplification?

### Lecture

Imagine trying to hear someone whispering in a crowded room.

Even though the person is speaking, their voice is so weak that it is easily overwhelmed by surrounding noise.

Neural signals present a similar challenge.

The electrical activity produced by neurons or muscles is very small. These signals must travel through biological tissue, electrodes, connecting cables, and electronic circuits before reaching the recording system. During this journey, they are easily affected by environmental interference and electronic noise.

If these signals are not strengthened before further processing, important physiological information may be lost.

Therefore, the first electronic component after the electrode is the amplifier.

### Key Definition

Amplification is the process of increasing the amplitude of a biological signal without altering the information it contains.

## 2.3.4 What is an Amplifier?

### Lecture

An amplifier is an electronic circuit designed to increase the strength of a weak electrical signal.

It is important to understand that an amplifier does not create new information.

Instead, it increases the magnitude of the existing signal so that it can be processed more accurately by the remaining components of the acquisition system.

For example, if an EEG signal has an amplitude of only a few microvolts, the amplifier increases this voltage to a level that electronic circuits can process reliably while preserving the original waveform.

A good amplifier should increase the desired signal while introducing as little additional noise or distortion as possible.

### Important Note

The purpose of amplification is not to improve the quality of the signal.

Its primary purpose is to increase the signal level so that subsequent processing stages can operate correctly.

# 2.3.5 Signal Conditioning

### Lecture

After amplification, the recorded signal still cannot be sent directly to the computer.

The signal may contain unwanted electrical interference, baseline drift, or other components that reduce recording quality.

Before digitisation, the signal is therefore prepared through a process known as signal conditioning.

Signal conditioning refers to the collection of operations that prepare the biological signal for accurate digital conversion.

Depending on the recording system, signal conditioning may include:

- Initial filtering

- Noise reduction

- Baseline stabilisation

- Impedance matching

- Signal isolation

The exact conditioning process depends on the recording modality and the design of the acquisition system.

### Key Definition

Signal conditioning refers to all operations performed on a biological signal to prepare it for accurate digitisation and analysis.

# 2.3.6 Amplification and Signal Conditioning in the Acquisition Chain

Let us return to the neural signal acquisition chain.

Biological Source → Electrode → Amplifier → Signal Conditioning → ADC → Computer

Once the electrode detects the biological signal, the amplifier increases its strength.

The conditioned signal is then cleaned and prepared for digitisation.

Only after these steps is the signal suitable for conversion into digital data by the Analog-to-Digital Converter.

Understanding this sequence is important because later stages of the acquisition chain depend heavily on the quality of the amplified and conditioned signal.

# 2.3.7 Practical Considerations

When designing a neural recording system, amplification and signal conditioning must be performed carefully.

Too little amplification may produce a signal that is difficult to analyse.

Excessive amplification, however, may also amplify unwanted noise or even saturate the recording system.

Similarly, poor signal conditioning may remove useful physiological information or allow unwanted interference to remain in the recording.

For this reason, amplification and signal conditioning are considered critical stages in neural signal acquisition.

## 2.3.8 Summary

In this section, we examined the role of amplification and signal conditioning in neural signal acquisition.

Neural signals are extremely weak and therefore require amplification before further processing.

Signal conditioning prepares the amplified signal for digitisation by reducing unwanted interference and ensuring that the signal entering the ADC is suitable for accurate digital conversion.

Together, amplification and signal conditioning improve the reliability of the recorded signal and support accurate downstream analysis.

## Transition to Section 2.4

At this stage, the neural signal has been detected, amplified, and conditioned.

However, it is still an analogue signal.

Since computers can process only digital information, the next step is to convert this continuous electrical signal into a sequence of numerical values.

In the next section, we will discuss sampling and Analog-to-Digital Conversion (ADC) and understand how analogue neural signals become digital data ready for computer analysis.