# 5.2 Low-Pass Filtering

Duration: ~10 Minutes

# 5.2.1 Introduction

In the previous section, we learned that filtering is used to improve the quality of neural recordings by reducing unwanted signal components. However, not all unwanted signals occupy the same frequency range. Some unwanted components occur at very high frequencies, while others occur at much lower frequencies.

To remove unwanted high-frequency components while preserving slower physiological activity, one of the most commonly used filters is the low-pass filter.

In this section, we will understand the operating principle of a low-pass filter, learn how it modifies a signal, and discuss its applications in neural signal processing.

# 5.2.2 Learning Objectives

After completing this section, you should be able to:

- Define a low-pass filter.

- Explain the operating principle of a low-pass filter.

- Describe the concept of cutoff frequency.

- Identify situations where low-pass filtering is useful.

- Understand the advantages and limitations of low-pass filtering.

# 5.2.3 What is a Low-Pass Filter?

### Definition

A low-pass filter is a filter that allows low-frequency components of a signal to pass while attenuating or removing high-frequency components above a selected cutoff frequency.

### Lecture

The name itself explains how the filter works.

The term low-pass means that low frequencies are allowed to pass through the filter, while higher frequencies are progressively reduced.

Imagine a gate that allows only slow-moving vehicles to pass while stopping fast-moving vehicles.

A low-pass filter behaves in a similar manner.

Instead of separating vehicles according to speed, it separates signal components according to their frequency.

Low-frequency components continue through the filter with little or no change, whereas high-frequency components are attenuated.

### Key Definition

Low-Pass Filter: A filter that preserves low-frequency components while reducing high-frequency components.

# 5.2.4 The Concept of Cutoff Frequency

A low-pass filter does not simply separate frequencies into two completely distinct groups.

Instead, it is designed around a parameter known as the cutoff frequency.

### Definition

The cutoff frequency is the frequency above which signal components begin to be attenuated by the filter.

### Lecture

Suppose a low-pass filter has a cutoff frequency of 40 Hz.

This means that frequency components below 40 Hz are allowed to pass with minimal attenuation.

As the frequency increases beyond 40 Hz, the filter gradually reduces the amplitude of those higher-frequency components.

It is important to understand that the cutoff frequency is not an abrupt boundary where signals are completely removed. Instead, attenuation increases progressively beyond the cutoff frequency.

(Insert Figure 5.2: Frequency response of a low-pass filter showing the passband, cutoff frequency, and attenuation region.)

# 5.2.5 How Does a Low-Pass Filter Affect a Signal?

Consider an EEG recording that contains both useful brain activity and unwanted high-frequency electrical noise.

When this recording passes through a low-pass filter:

- Low-frequency brain activity is preserved.

- High-frequency noise is reduced.

- The resulting waveform appears smoother and easier to interpret.

The filter therefore improves the visibility of the physiological signal by suppressing unwanted high-frequency components.

However, it is important to remember that the filter removes all frequencies above the selected cutoff, regardless of whether they represent noise or useful physiological information.

For this reason, the cutoff frequency must be selected carefully.

# 5.2.6 Applications in Neural Signal Processing

Low-pass filters are widely used in neuroscience because many physiological signals contain important information at relatively low frequencies.

Some common applications include:

### EEG

Low-pass filters are frequently used to reduce high-frequency electrical noise while preserving the frequency components relevant for EEG analysis.

### Local Field Potentials (LFP)

Since LFPs primarily contain lower-frequency activity, low-pass filtering is commonly used to separate LFPs from higher-frequency spike activity.

### EMG Envelope Extraction

After rectifying an EMG signal, a low-pass filter is often applied to obtain the smooth EMG envelope, which represents the overall level of muscle activation.

# 5.2.7 Advantages

Low-pass filtering offers several advantages.

- Reduces unwanted high-frequency noise.

- Produces smoother waveforms.

- Improves signal readability.

- Serves as an important preprocessing step before feature extraction.

# 5.2.8 Limitations

Although low-pass filtering is extremely useful, it must be applied carefully.

If the cutoff frequency is selected too low, useful physiological information may also be removed.

Excessive low-pass filtering can make the signal appear overly smooth and may distort rapidly changing neural events.

Therefore, the cutoff frequency should always be chosen according to the characteristics of the signal being analysed.

### Important Note

A low-pass filter removes high-frequency components, not necessarily noise.

If useful physiological information exists above the selected cutoff frequency, that information will also be attenuated.

This is why filter selection should always be guided by the characteristics of the biological signal rather than by convenience alone.

# 5.2.9 Summary

In this section, we introduced the low-pass filter and examined its role in neural signal processing.

We learned that a low-pass filter allows low-frequency components to pass while reducing frequencies above a selected cutoff frequency.

Low-pass filtering is widely used to suppress high-frequency interference and improve signal quality. However, careful selection of the cutoff frequency is essential to avoid removing useful physiological information.

## Transition to Section 5.3

The low-pass filter removes high-frequency components while preserving lower frequencies.

But in some situations, the opposite is required.

Certain recordings contain unwanted low-frequency components, such as baseline drift or slow movement artefacts, while the useful physiological information lies at higher frequencies.

To address this problem, we use another fundamental filter—the High-Pass Filter.