# 5.3 High-Pass Filtering

Duration: ~10 Minutes

# 5.3.1 Introduction

In the previous section, we learned that a low-pass filter preserves low-frequency components while reducing high-frequency components. However, not all unwanted signals occur at high frequencies.

Many neural recordings contain slow fluctuations that are not part of the physiological signal of interest. These slow variations may arise from electrode movement, respiration, changes in electrode impedance, or gradual shifts in the recording baseline. Such low-frequency components can obscure important neural activity and make subsequent analysis more difficult.

To reduce these unwanted slow components while preserving higher-frequency information, we use a high-pass filter.

In this section, we will examine how a high-pass filter works, understand the concept of cutoff frequency in high-pass filtering, and discuss its applications in neural signal processing.

# 5.3.2 Learning Objectives

After completing this section, you should be able to:

- Define a high-pass filter.

- Explain how a high-pass filter operates.

- Describe the role of cutoff frequency.

- Identify applications of high-pass filtering in neuroscience.

- Understand the advantages and limitations of high-pass filtering.

# 5.3.3 What is a High-Pass Filter?

### Definition

A high-pass filter is a filter that allows high-frequency components of a signal to pass while attenuating or removing low-frequency components below a selected cutoff frequency.

### Lecture

As its name suggests, a high-pass filter performs the opposite operation of a low-pass filter.

Instead of preserving slow signal variations, it removes them and allows faster signal variations to remain.

Imagine a gate that allows only fast-moving vehicles to pass while stopping slower ones.

Similarly, a high-pass filter allows high-frequency signal components to continue through the system while reducing lower-frequency components.

This makes the filter particularly useful when unwanted information exists primarily at low frequencies.

### Key Definition

High-Pass Filter: A filter that preserves high-frequency components while attenuating low-frequency components below a selected cutoff frequency.

# 5.3.4 The Concept of Cutoff Frequency

Like the low-pass filter, the high-pass filter is also defined by a cutoff frequency.

### Definition

The cutoff frequency is the frequency below which signal components begin to be attenuated by the filter.

### Lecture

Suppose a high-pass filter has a cutoff frequency of 1 Hz.

Signal components above 1 Hz are largely preserved.

Components below 1 Hz are progressively attenuated.

Again, this transition is gradual rather than abrupt. Frequencies close to the cutoff are only partially attenuated, while frequencies much lower than the cutoff experience much greater attenuation.

(Insert Figure 5.3: Frequency response of a high-pass filter showing the stopband, cutoff frequency, and passband.)

# 5.3.5 How Does a High-Pass Filter Affect a Signal?

Consider an EEG recording containing slow baseline drift caused by gradual changes in electrode contact.

Although the neural activity itself may be present, the drifting baseline makes the waveform difficult to interpret.

When a high-pass filter is applied:

- Slow baseline drift is reduced.

- The baseline becomes more stable.

- Faster neural activity becomes easier to observe.

The filter therefore improves the visibility of the physiological signal by reducing unwanted low-frequency components.

However, the filter cannot distinguish between useful and unwanted low-frequency information. Any physiological activity occurring below the selected cutoff frequency will also be attenuated.

For this reason, the cutoff frequency must always be selected carefully.

# 5.3.6 Applications in Neural Signal Processing

High-pass filters are widely used in biomedical signal processing to remove slow variations that interfere with signal interpretation.

### EEG

High-pass filters help reduce slow baseline drift caused by electrode movement or changes in electrode impedance, making EEG recordings more stable for analysis.

### EMG

EMG signals often contain low-frequency motion artefacts produced by body movement.

High-pass filtering reduces these slow artefacts while preserving the higher-frequency components associated with muscle activity.

### Spike Recordings

Extracellular recordings from microelectrodes often use high-pass filters to isolate action potentials from slower local field potentials.

Since spikes contain predominantly high-frequency components, high-pass filtering helps separate them from slower neural activity.

# 5.3.7 Advantages

High-pass filtering provides several important benefits.

- Reduces baseline drift.

- Suppresses slow movement artefacts.

- Improves baseline stability.

- Enhances the visibility of rapid physiological events.

- Serves as an important preprocessing step before spike detection and EMG analysis.

# 5.3.8 Limitations

Although high-pass filtering is useful, it should be applied with care.

If the cutoff frequency is selected too high, slow physiological components that are important for analysis may also be removed.

For example, excessively high cutoff frequencies can distort slow EEG activity or alter the shape of event-related potentials.

Therefore, the cutoff frequency should always be selected according to the characteristics of the recorded signal and the objectives of the analysis.

### Important Note

A high-pass filter removes low-frequency components, not simply noise.

If useful physiological information exists below the selected cutoff frequency, that information will also be attenuated.

Filter settings should therefore be chosen based on the characteristics of the biological signal rather than using a fixed value for every application.

# 5.3.9 Summary

In this section, we introduced the high-pass filter and examined its role in neural signal processing.

We learned that a high-pass filter preserves high-frequency components while reducing frequencies below a selected cutoff frequency.

High-pass filtering is commonly used to reduce baseline drift, suppress slow movement artefacts, and improve the visibility of rapidly changing neural activity. Like all filters, it must be configured carefully to avoid removing useful physiological information.

## Transition to Section 5.4

So far, we have studied two fundamental filters.

A low-pass filter preserves low frequencies, while a high-pass filter preserves high frequencies.

However, many neural signals occupy only a specific range of frequencies. In such situations, we may wish to remove both very low and very high frequencies while preserving only the frequencies of interest.

To accomplish this, we use a band-pass filter, which will be the focus of the next section.