# 5.5 Notch Filtering

Duration: ~7 Minutes

# 5.5.1 Introduction

In the previous sections, we learned about low-pass, high-pass, and band-pass filters. These filters remove broad ranges of frequencies to improve the quality of neural recordings.

However, not all unwanted signals occupy a wide frequency range.

Some types of interference occur almost entirely at a single frequency. One of the most common examples is power-line interference, which appears at 50 Hz in many countries and 60 Hz in others.

Removing an entire frequency range to eliminate this interference would also remove useful physiological information. Instead, we need a filter that removes only a very narrow frequency band while leaving the rest of the signal largely unchanged.

This filter is known as the notch filter.

# 5.5.2 Learning Objectives

After completing this section, you should be able to:

- Define a notch filter.

- Explain how a notch filter works.

- Describe why notch filters are used in neural signal processing.

- Identify common applications of notch filtering.

- Understand the advantages and limitations of notch filters.

# 5.5.3 What is a Notch Filter?

### Definition

A notch filter is a filter that attenuates a very narrow range of frequencies while allowing frequencies above and below that range to pass with minimal attenuation.

### Lecture

Unlike the filters we have studied so far, a notch filter is designed to remove only a specific unwanted frequency.

Imagine listening to music while a single annoying tone is constantly present in the background.

Instead of lowering the volume of the entire recording, it would be much better to remove only that unwanted tone.

A notch filter works in exactly this way.

It targets a very narrow frequency band while preserving almost all other components of the signal.

For this reason, notch filters are particularly useful when the unwanted interference occurs at a known frequency.

### Key Definition

Notch Filter: A filter that removes a narrow band of frequencies centred around a specific frequency while preserving the remaining signal.

# 5.5.4 Why is a Notch Filter Needed?

### Lecture

One of the most common sources of interference during biomedical recordings is the electrical power supply.

Electrical devices, laboratory equipment, lighting systems, and power cables all generate electromagnetic fields.

These fields can couple into recording electrodes and appear in the recorded signal as power-line interference.

The frequency of this interference depends on the local electrical power system.

- 50 Hz in many countries.

- 60 Hz in others.

Because the interference occurs at a well-defined frequency, it can often be reduced effectively using a notch filter centred at that frequency.

Rather than removing a broad range of frequencies, the notch filter removes only the narrow frequency band associated with the electrical interference.

### Figure 5.5

Signal Spectrum → 50 Hz Peak → After Notch Filter

(Insert a frequency spectrum illustrating a sharp 50 Hz interference peak before filtering and its reduction after notch filtering.)

# 5.5.5 How Does a Notch Filter Affect a Signal?

Consider an EEG recording contaminated by 50 Hz power-line interference.

Before filtering, the recording contains both the neural signal and a strong electrical component at 50 Hz.

When a notch filter centred at 50 Hz is applied:

- The narrow frequency band around 50 Hz is attenuated.

- Most of the remaining frequency components are preserved.

- The influence of power-line interference is significantly reduced.

As a result, the neural signal becomes easier to interpret without substantially altering frequencies outside the rejected band.

# 5.5.6 Applications in Neural Signal Processing

Notch filters are commonly used whenever recordings are contaminated by electrical interference.

### EEG

Notch filters are frequently applied to reduce 50 Hz or 60 Hz power-line interference that may contaminate scalp recordings.

### EMG

Surface EMG recordings can also pick up electrical interference from nearby equipment. A notch filter helps suppress this unwanted component while preserving most of the muscle activity.

### ECoG and Local Field Potentials

Although invasive recordings generally have better signal quality, they can still be affected by power-line interference. In such cases, notch filtering is used as part of the preprocessing pipeline.

# 5.5.7 Advantages

Notch filtering offers several advantages.

- Effectively removes narrow-band interference.

- Preserves most of the useful physiological signal.

- Simple to implement.

- Widely used in biomedical signal processing.

# 5.5.8 Limitations

Although notch filters are highly effective for removing power-line interference, they should not be used unnecessarily.

If useful physiological information exists near the notch frequency, that information may also be attenuated.

Furthermore, a notch filter removes only a specific narrow frequency band. It is not designed to remove broadband noise or motion artefacts.

Therefore, notch filtering should be used only when there is clear evidence of narrow-band interference.

### Important Note

A notch filter is not a replacement for proper recording practices.

Whenever possible, power-line interference should first be minimised by ensuring good electrode contact, proper grounding, appropriate shielding, and correct equipment setup.

Filtering should be used to reduce residual interference rather than compensate for poor recording conditions.

# 5.5.9 Summary

In this section, we introduced the notch filter and examined its role in neural signal processing.

We learned that a notch filter removes a very narrow frequency band centred around a specific frequency, making it particularly useful for reducing power-line interference at 50 Hz or 60 Hz.

Because notch filters affect only a small portion of the frequency spectrum, they preserve most of the physiological signal while reducing unwanted electrical interference.

## Transition to Section 5.6

The filters we have studied so far all separate signals based on their frequency content.

However, not every filtering technique works in the frequency domain.

Some filters operate directly on the signal waveform in the time domain, making them particularly useful for reducing random fluctuations and impulsive noise.

In the next section, we will study two simple yet widely used time-domain filters: the moving-average filter and the median filter.