# 6.2 Short-Time Fourier Transform (STFT)

Duration: ~8 Minutes

# 6.2.1 Introduction

In the previous section, we learned that neural signals are generally nonstationary. Their frequency content changes continuously as neural activity evolves over time.

We also identified an important limitation of the conventional Fourier Transform.

Although it tells us which frequencies are present in a signal, it cannot tell us when those frequencies occur.

For many neuroscience applications, this information is essential.

Imagine analysing an EEG recording during a visual experiment.

Suppose alpha activity appears only after the participant closes their eyes.

The Fourier Transform can indicate that alpha frequencies exist somewhere in the recording, but it cannot identify the exact moment at which they appeared.

To solve this problem, we need a method that analyses both time and frequency simultaneously.

One of the earliest and most widely used solutions is the Short-Time Fourier Transform, commonly abbreviated as STFT.

# 6.2.2 Learning Objectives

After completing this section, you should be able to:

Explain why the Short-Time Fourier Transform was developed.

Describe the basic principle of STFT.

Understand the concept of a moving analysis window.

Explain how STFT provides both time and frequency information.

Recognise the advantages and limitations of STFT.

# 6.2.3 Why Do We Need STFT?

### Lecture

Let us begin by revisiting the problem.

Suppose you record an EEG signal for thirty seconds.

During the first ten seconds, alpha activity is dominant.

During the next ten seconds, beta activity becomes stronger.

Finally, during the last ten seconds, gamma activity appears briefly.

If we apply a conventional Fourier Transform to the entire thirty-second recording, the result is a single frequency spectrum representing the entire signal.

Although this spectrum tells us that alpha, beta, and gamma frequencies are present, it provides no information about when each frequency occurred.

This is a significant limitation because neural activity is dynamic.

In neuroscience, timing is often just as important as frequency.

To understand brain function, we need a method that tells us both:

Which frequencies are present?

At what time do they occur?

This is precisely the problem that the Short-Time Fourier Transform was designed to solve.

### Important Note

The Fourier Transform answers "What frequencies exist?"

The Short-Time Fourier Transform answers "What frequencies exist, and when do they occur?"

# 6.2.4 What is the Short-Time Fourier Transform?

### Definition

The Short-Time Fourier Transform (STFT) is a time–frequency analysis technique that divides a signal into short time segments and performs a Fourier Transform on each segment separately.

### Lecture

Rather than analysing the entire signal at once, STFT analyses only a small portion of the signal at a time.

Imagine placing a small window over the beginning of the signal.

Only the data inside this window are analysed.

The window is then moved slightly forward, and the analysis is repeated.

This process continues until the entire signal has been examined.

Each window produces its own frequency spectrum.

When these spectra are combined, they reveal how the frequency content changes throughout the recording.

This simple idea transforms the Fourier Transform from a purely frequency-based analysis into a time-frequency analysis.

### Figure 6.2

Original Signal → Sliding Window → Fourier Transform for Each Window → Time-Frequency Representation

(Insert a diagram illustrating a sliding window moving across a signal, with a Fourier Transform applied to each window.)

# 6.2.5 How Does STFT Work?

The operation of STFT can be understood in four simple steps.

### Step 1 — Select a Short Time Window

A small portion of the signal is selected using a window function.

Only the data within this window are analysed.

### Step 2 — Apply the Fourier Transform

A Fourier Transform is performed on the data inside the window.

This provides the frequency content for that short time interval.

### Step 3 — Move the Window

The window is shifted slightly along the signal.

A new portion of the signal is selected.

### Step 4 — Repeat the Process

The Fourier Transform is computed again for the new window.

Repeating this process across the entire recording produces a sequence of frequency spectra describing how the signal changes over time.

# 6.2.6 Window Size and the Time–Frequency Trade-off

One of the most important concepts in STFT is the window size.

The size of the analysis window determines how the signal is examined.

### Small Window

A smaller window provides better time resolution because it can detect rapid changes in the signal.

However, it contains fewer data points, reducing frequency resolution.

### Large Window

A larger window contains more data points and therefore provides better frequency resolution.

However, it cannot localise rapid changes in time as accurately.

### Important Concept

This creates a fundamental trade-off.

A small window tells us when something happened more accurately.

A large window tells us which frequency is present more accurately.

Unfortunately, a single fixed window cannot optimise both simultaneously.

(Insert Figure 6.3: Comparison of small and large STFT windows showing the trade-off between time resolution and frequency resolution.)

### Important Note

STFT uses a fixed window size.

This means that the same window is used throughout the entire signal, regardless of whether the signal changes rapidly or slowly.

As we will see later, this fixed-window approach is one of the major limitations of STFT.

# 6.2.7 Applications in Neural Signal Processing

The Short-Time Fourier Transform is widely used because many neural signals change continuously over time.

Some common applications include:

### EEG

Tracking how neural oscillations change during cognitive tasks, sleep, or sensory stimulation.

### EMG

Observing changes in muscle activity during movement and fatigue.

### Event-Related Neural Activity

Examining frequency changes before, during, and after stimulus presentation.

### Brain–Computer Interfaces

Monitoring time-varying neural rhythms used for feature extraction and classification.

# 6.2.8 Advantages

The Short-Time Fourier Transform provides several important advantages.

Describes both time and frequency.

Suitable for analysing nonstationary signals.

Relatively simple to implement.

Forms the basis for spectrogram generation.

# 6.2.9 Limitations

Although STFT represents a major improvement over the conventional Fourier Transform, it is not a perfect solution.

Because it uses a single fixed window, it cannot simultaneously achieve high time resolution and high frequency resolution.

This limitation becomes particularly important when analysing neural signals that contain both slow oscillations and rapid transient events.

For this reason, more advanced techniques, such as wavelet analysis, were later developed.

# 6.2.10 Summary

In this section, we introduced the Short-Time Fourier Transform (STFT).

We learned that STFT overcomes one of the major limitations of the conventional Fourier Transform by analysing short segments of the signal instead of the entire recording.

This allows us to determine both the frequency content and its variation over time.

However, we also learned that STFT uses a fixed analysis window, creating a trade-off between time resolution and frequency resolution.

## Transition to Section 6.3

The output of the Short-Time Fourier Transform consists of a sequence of frequency spectra, one for each time window.

While this information is valuable, examining hundreds of separate spectra is neither practical nor intuitive.

Instead, we need a way to visualise how frequency changes over time in a single image.

This visual representation is known as a spectrogram, and it will be the focus of the next section.